+--------------------------------------------------------------------+

|               Instructions sorted by Z80 mnemonics (compact)       |

+-----------+--------------+---------------+----------------+--------+

|  Op-codes |              |               |   Operation,   | Flags  |

+--------+--+ Z80 Assembly | R800 Assembly |  Description   |        |

| Binary |Hx|  Mnemonics   |   Mnemonics   |  and Remarks   +--------+

|76543210|  |  (meaning)   |   (meaning)   |                |SZ.HP.NC|

+========+==+==============+===============+================+========+

|00ss1001|  |add hl,ss     |add .hl,ss     |.hl <- .hl + ss |**?*o?0*|

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|add ii,pp     |add ii,pp      |ii <- ii + pp   |**?*o?0*|

|00pp1001|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10000 r |  |add a,r       |add .a,r       |.a <- .a + r    |**?*o?0*|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |add .a,p       |.a <- .a + p    |**?*o?0*|

|10000 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |add .a,q       |.a <- .a + q    |**?*o?0*|

|10000 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10000110|86|add a,(hl)    |add .a,[.hl]   |.a <- .a + [.hl]|**?*o?0*|

|        |  |(ADD)         |(ADD)          |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|add a,(ii+d)  |add .a,[ii+d]  |.a <- .a+[ii+d] |**?*o?0*|

|10000110|86|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10001 r |  |adc a,r       |addc .a,r      |.a <- .a + r + C|**?*o?0*|

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |addc .a,q      |.a <- .a + q + C|**?*o?0*|

|10001 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |addc .a,p      |.a <- .a + p + C|**?*o?0*|

|10001 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10001110|8E|adc a,(hl)    |addc .a,[.hl]  |.a <- .a+[.hl]+C|**?*o?0*|

|        |  |(ADd with     |(ADD with      |                |        |

|        |  |        Carry)|         Carry)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|adc a,(ii+d)  |addc .a,[ii+d] |.a<-.a+[ii+d]+C |**?*o?0*|

|10001110|8E|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10100 r |  |and r         |and .a,r       |.a <- .a & r    |**?1p?00|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |and .a,p       |.a <- .a & p    |**?1p?00|

|10100 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |and .a,q       |.a <- .a & q    |**?1p?00|

|10100 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10100110|A6|and (hl)      |and .a,[.hl]   |.a <- .a & [.hl]|**?1p?00|

|        |  |(AND)         |(AND)          |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|and (ii+d)    |and .a,[ii+d]  |.a<-.a & [ii+d] |**?1p?00|

|10100110|A6|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11000110|C6|add a,n       |add .a,n       |.a <- .a + n    |**?*o?0*|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001110|CE|adc a,n       |addc .a,n      |.a <- .a + n + C|**?*o?0*|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11100110|E6|and n         |and .a,n       |.a <- .a & n    |**?1p?00|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|adc hl,ss     |addc .hl,ss    |.hl <- .hl+ss+C |**?*o?0*|

|01ss1010|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|bit b,r       |bit b,r        |Z <- ~ r{b}     |?*?1??0.|

|01 b  r |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|bit b,(ii+d)  |bit b,[ii+d]   |Z <- ~ [ii+d]{b}|?*?1??0.|

|11001011|CB|              |               |                |        |

|   d    |d |              |               |                |        |

|01 b 110|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|bit b,(hl)    |bit b,[.hl]    |Z <- ~ [.hl]{b} |?*?1??0.|

|01 b 110|  |(test BIT)    |(test BIT)     |                |        |

+--------+--+--------------+---------------+----------------+--------+

**********************

|11001101|CD|call nn       |call nn        |[.sp-2] <- .pcl |........|

|  nnl   |  |(CALL)        |(CALL)         |[.sp-1] <- .pch |        |

|  nnh   |  |              |               |.sp <- .sp - 2  |        |

|        |  |              |               |.pc <- nn       |        |

+--------+--+--------------+---------------+----------------+--------+

|11011100|DC|call c,nn     |call c,nn      |if C = 1        |........|

|  nnl   |  |(CALL if      |(CALL if Carry)| [.sp-2] <- .pcl|        |

|  nnh   |  |        Carry)|               | [.sp-1] <- .pch|        |

|        |  |              |               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11111100|FC|call m,nn     |call m,nn      |if S = 1        |........|

|  nnl   |  |(CALL if      |(CALL if Minus)| [.sp-2] <- .pcl|        |

|  nnh   |  |        Minus)|               | [.sp-1] <- .pch|        |

|        |  |              |               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11010100|D4|call nc,nn    |call nc,nn     |if C = 0        |........|

|  nnl   |  |(CALL if      |(CALL if       | [.sp-2] <- .pcl|        |

|  nnh   |  |    Not Carry)|     Not Carry)| [.sp-1] <- .pch|        |

|        |  |              |               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11000100|C4|call nz,nn    |call nz,nn     |if Z = 0        |........|

|  nnl   |  |(CALL if Not  |(CALL if       | [.sp-2] <- .pcl|        |

|  nnh   |  |     Not Zero)|      Not Zero)| [.sp-1] <- .pch|        |

|        |  |              |               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11110100|F4|call p,nn     |call p,nn      |if S = 0        |........|

|  nnl   |  |(CALL if Plus)|(CALL if Plus) | [.sp-2] <- .pcl|        |

|  nnh   |  |              |               | [.sp-1] <- .pch|        |

|        |  |              |               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11101100|EC|call pe,nn    |call pe,nn     |if P = 1        |........|

|  nnl   |  |(CALL if      |(CALL if Parity| [.sp-2] <- .pcl|        |

|  nnh   |  |  Parity is   |       is Even)| [.sp-1] <- .pch|        |

|        |  |         Even)|               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11100100|E4|call po,nn    |call po,nn     |if P = 0        |........|

|  nnl   |  |(CALL if      |(CALL if Parity| [.sp-2] <- .pcl|        |

|  nnh   |  |Parity is Odd)|        is Odd)| [.sp-1] <- .pch|        |

|        |  |              |               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11001100|CC|call z,nn     |call z,nn      |if Z = 1        |........|

|  nnl   |  |(CALL if Zero)|(CALL if Zero) | [.sp-2] <- .pcl|        |

|  nnh   |  |              |               | [.sp-1] <- .pch|        |

|        |  |              |               | .sp <- .sp - 2 |        |

|        |  |              |               | .pc <- nn      |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|00111111|3F|ccf           |notc           |C <- ~ C        |..??.?0*|

|        |  |(Complement   |(NOT Carry)    |                |        |

|        |  |   Carry Flag)|               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10111110|BE|cp (hl)       |cmp .a,[.hl]   |.a - [.hl]      |**?*o?1*|

|        |  |(ComPare)     |(CoMPare)      |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|cp (ii+d)     |cmp .a,[ii+d]  |.a - [ii+d]     |**?*o?1*|

|10111110|BE|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111110|FE|cp n          |cmp .a,n       |.a - n          |**?*o?1*|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10111 r |  |cp r          |cmp .a,r       |.a - r          |**?*o?1*|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |cmp .a,p       |.a - p          |**?*o?1*|

|10111 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |cmp .a,q       |.a - q          |**?*o?1*|

|10111 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|cpd           |cmp .a,[.hl--] |.a - [.hl]      |*%?*$?*.|

|10101001|A9|(ComPare and  |               |.hl <- .hl - 1  |        |

|        |  |    Decrement)|               |.bc <- .bc - 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|cpdr          |cmpm .a,[.hl--]|repeat          |*%?*$?*.|

|10111001|B9|(ComPare,     |               |  .a - [.hl]    |        |

|        |  |  Decrement   |               |  .hl <- .hl - 1|        |

|        |  |   and Repeat)|               |  .bc <- .bc - 1|        |

|        |  |              |               |until .bc = 0 or|        |

|        |  |              |               |      .a = [.hl]|        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|cpi           |cmp .a,[.hl++] |.a - [.hl]      |*%?*$?*.|

|10100001|A1|(ComPare and  |               |.hl <- .hl + 1  |        |

|        |  |    Increment)|               |.bc <- .bc - 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|cpir          |cmpm .a,[.hl++]|repeat          |*%?*$?*.|

|10110001|B1|(ComPare,     |(CoMPare       |  .a - [.hl]    |        |

|        |  |  Increment   |      Multiple)|  .hl <- .hl + 1|        |

|        |  |   and Repeat)|               |  .bc <- .bc - 1|        |

|        |  |              |               |until .bc = 0 or|        |

|        |  |              |               |      .a = [.hl]|        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|00101111|2F|cpl           |not .a         |.a <- ~ .a      |..?1.?1.|

|        |  |(ComPLement   |(NOT)          |                |        |

|        |  |  Accumulator)|               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00100111|27|daa           |adj .a         |adjust to       |**?*p?.*|

|        |  |(Decimal      |(ADJust)       |         decimal|        |

|        |  |   Adjust     |               |                |        |

|        |  |  Accumulator)|               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00110101|35|dec (hl)      |dec [.hl]      |[.hl] <- [.hl]-1|**?*o?0*|

|        |  |(DECrement)   |(DECrement)    |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|dec (ii+d)    |dec [ii+d]     |[ii+d]<-[ii+d]-1|**?*o?0*|

|00110101|35|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00 r 101|  |dec r         |dec r          |r <- r - 1      |**?*o?0*|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |dec p          |p <- p - 1      |**?*o?0*|

|00 p 101|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |dec q          |q <- q - 1      |**?*o?0*|

|00 q 101|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00ss1011|  |dec ss        |dec ss         |ss <- ss - 1    |........|

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|dec ii        |dec ii         |ii <- ii - 1    |........|

|00101011|2B|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11110011|F3|di            |di             |IFF <- 0        |........|

|        |  |(Disable      |(Disable       |                |        |

|        |  | Interruption)|  Interruption)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|00010000|10|djnz d        |dbnz d         |.b <- .b - 1    |........|

|   d    |  |(Decrement    |(Decrement     |if .b not zero  |        |

|        |  | and Jump if  | and Branch if |   pc <- .pc + d|        |

|        |  |     Not Zero)|      Not Zero)|else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11111011|FB|ei            |ei             |IFF <- 1        |........|

|        |  |(Enable       |(Enable        |                |        |

|        |  | Interruption)|  Interruption)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11100011|E3|ex (sp),hl    |xch [.sp],.hl  |.l <-> [.sp]    |........|

|        |  |(EXchange)    |(eXCHange)     |.h <-> [.sp+1]  |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ex (sp),ii    |xch [.sp],ii   |iil <-> [.sp]   |........|

|11100011|E3|              |               |iih <-> [.sp+1] |        |

+--------+--+--------------+---------------+----------------+--------+

|00001000|08|ex af,af'     |xch .af,.af'   |.af <-> .af'    |........|

+--------+--+--------------+---------------+----------------+--------+

|11101011|EB|ex de,hl      |xch .de,.hl    |.de <-> .hl     |........|

+--------+--+--------------+---------------+----------------+--------+

|11011001|D9|exx           |xchx           |.bc <-> .bc'    |........|

|        |  |(EXchange with|(eXCHange with |.de <-> .de'    |        |

|        |  |     auXiliar)|      auXiliar)|.hl <-> .hl'    |        |

+--------+--+--------------+---------------+----------------+--------+

|01110110|76|halt          |halt           |halt processor  |........|

|        |  |(HALT)        |(HALT)         |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|im 0          |im 0           |set interruption|........|

|01000110|46|(Interruption |(Interruption  |mode 0          |        |

|        |  |       Mode 0)|        Mode 0)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|im 1          |im 1           |set interruption|........|

|01010110|56|(Interruption |(Interruption  |mode 1          |        |

|        |  |       Mode 1)|        Mode 1)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|im 2          |im 2           |set interruption|........|

|01011110|5E|(Interruption |(Interruption  |mode 2          |        |

|        |  |       Mode 2)|        Mode 2)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11011011|DB|in a,(n)      |in .a,[n]      |.a <- [n]       |........|

|   n    |n |(INput)       |(INput)        |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|in r,(c)      |in r,[.c]      |r <- [.c]       |**?0p?0.|

|01 r 000|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED| *1           |in .f,[.c]     |[.c]            |**?0p?0.|

|01110000|70|              |               |* note 1: this  |        |

|        |  |              |               |instruction has |        |

|        |  |              |               |no mnemonic for |        |

|        |  |              |               |Z80, so its     |        |

|        |  |              |               |op-code must be |        |

|        |  |              |               |directly defined|        |

|        |  |              |               |when necessary  |        |

|        |  |              |               |(in pseudo-code:|        |

|        |  |              |               |db 0edh,070h)   |        |

|        |  |              |               |* note 2: the   |        |

|        |  |              |               |R800 mnemonic   |        |

|        |  |              |               |implies, but    |        |

|        |  |              |               |doesn't mean,   |        |

|        |  |              |               |that the datum  |        |

|        |  |              |               |is stored in .f;|        |

|        |  |              |               |the mnemonic    |        |

|        |  |              |               |only indicates  |        |

|        |  |              |               |the instruction |        |

|        |  |              |               |affects .f      |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ind           |in             |[.hl] <- in[.c] |?!????1.|

|10101010|AA|(INput and    |   [.hl--],[.c]|.b <- .b - 1    |        |

|        |  |    Decrement)|(INput)        |.hl <- .hl - 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|indr          |inm            |repeat          |?1????1.|

|10111010|BA|(INput,       |   [.hl--],[.c]| [.hl] <- in[.c]|        |

|        |  |Decrement and |(INput         | .b <- .b - 1   |        |

|        |  |       Repeat)|      Multiple)| .hl <- .hl - 1 |        |

|        |  |              |               |until .b = 0    |        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ini           |in             |[.hl] <- in[.c] |?!????1.|

|10100010|A2|(INput and    |   [.hl++],[.c]|.b <- .b - 1    |        |

|        |  |    Increment)|(INput)        |.hl <- .hl + 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|inir          |inm            |repeat          |?1????1.|

|10110010|B2|(INput,       |   [.hl++],[.c]| [.hl] <- in[.c]|        |

|        |  |Increment and |(INput         | .b <- .b - 1   |        |

|        |  |       Repeat)|      Multiple)| .hl <- .hl + 1 |        |

|        |  |              |               |until .b = 0    |        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|00110100|34|inc (hl)      |inc [.hl]      |[.hl] <- [.hl]+1|**?*o?0*|

|        |  |(INCrement)   |(INCrement)    |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|inc (ii+d)    |inc [ii+d]     |[ii+d]<-[ii+d]+1|**?*o?0*|

|00110100|34|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00 r 100|  |inc r         |inc r          |r <- r + 1      |**?*o?0*|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |inc p          |p <- p + 1      |**?*o?0*|

|00 p 100|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |inc q          |q <- q + 1      |**?*o?0*|

|00 q 100|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00ss0011|  |inc ss        |inc ss         |ss <- ss + 1    |........|

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|inc ii        |inc ii         |ii <- ii + 1    |........|

|00100011|23|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101001|E9|jp (hl)       |br [.hl]       |.pc <- [.hl]    |........|

|        |  |(JumP)        |(BRanch)       |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|jp (ii)       |br [ii]        |.pc <- [ii]     |........|

|11101001|E9|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11000011|C3|jp nn         |br nn          |.pc <- nn       |........|

|  nnl   |  |              |               |       .pc <- nn|        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11011010|DA|jp c,nn       |bc nn          |if C = 1        |........|

|  nnl   |  |(JumP if      |(Branch if     |       .pc <- nn|        |

|  nnh   |  |        Carry)|         Carry)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111010|FA|jp m,nn       |bm nn          |if S = 1        |........|

|  nnl   |  |(JumP if      |(Branch if     |       .pc <- nn|        |

|  nnh   |  |        Minus)|         Minus)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11010010|D2|jp nc,nn      |bnc nn         |if C = 0        |........|

|  nnl   |  |(JumP if      |(Branch if     |       .pc <- nn|        |

|  nnh   |  |    Not Carry)|     Not Carry)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11000010|C2|jp nz,nn      |bnz nn         |if Z = 0        |........|

|  nnl   |  |(JumP if      |(Branch if     |       .pc <- nn|        |

|  nnh   |  |     Not Zero)|      Not Zero)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11110010|F2|jp p,nn       |bp nn          |if S = 0        |........|

|  nnl   |  |(JumP if Plus)|(Branch if     |       .pc <- nn|        |

|  nnh   |  |              |          Plus)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101010|EA|jp pe,nn      |bpe nn         |if P = 1        |........|

|  nnl   |  |(JumP if      |(Branch if     |       .pc <- nn|        |

|  nnh   |  |   Parity is  |Parity is Even)|                |        |

|        |  |         Even)|               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11100010|E2|jp po,nn      |bpo nn         |if P = 0        |........|

|  nnl   |  |(JumP if      |(Branch if     |       .pc <- nn|        |

|  nnh   |  |Parity is Odd)| Parity is Odd)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001010|CA|jp z,nn       |bz nn          |if Z = 1        |........|

|  nnl   |  |(JumP if Zero)|(Branch if     |       .pc <- nn|        |

|  nnh   |  |              |          Zero)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|00011000|18|jr d          |short br d     |.pc <- .pc + d  |........|

|   d    |  |(Jump         |(SHORT BRanch) |                |        |

|        |  |     Relative)|               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00111000|38|jr c,d        |short bc d     |if C = 1        |........|

|   d    |  |(Jump         |(SHORT Branch  |  .pc <- .pc + d|        |

|        |  | Relative if  |      if Carry)|else            |        |

|        |  |        Carry)|               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|00110000|30|jr nc,d       |short bnc d    |if C = 0        |........|

|   d    |  |(Jump         |(SHORT Branch  |  .pc <- .pc + d|        |

|        |  | Relative if  |  if Not Carry)|else            |        |

|        |  |    Not Carry)|               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|00100000|20|jr nz,d       |short bnz d    |if Z = 0        |........|

|   d    |  |(Jump         |(SHORT Branch  |  .pc <- .pc + d|        |

|        |  | Relative if  |   if Not Zero)|else            |        |

|        |  |     Not Zero)|               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|00101000|28|jr z,d        |short bz d     |if Z = 1        |........|

|   d    |  |(Jump         |(SHORT Branch  |  .pc <- .pc + d|        |

|        |  | Relative if  |       if Zero)|else            |        |

|        |  |         Zero)|               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|00110010|32|ld (nn),a     |ld [nn],.a     |[nn] <- .a      |........|

|  nnl   |  |(LoaD)        |(LoaD)         |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ld (nn),ss    |ld [nn],ss     |[nn+1] <- ssh   |........|

|01ss0011|  |              |               |[nn] <- ssl     |        |

|  nnl   |  |              |               |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00100010|22|ld (nn),hl    |ld [nn],.hl    |[nn+1] <- .h    |........|

|  nnl   |  |              |               |[nn] <- .l      |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ld (nn),ii    |ld [nn],ii     |[nn+1] <- iih   |........|

|00100010|22|              |               |[nn] <- iil     |        |

|  nnl   |  |              |               |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00000010|02|ld (bc),a     |ld [.bc],.a    |[.bc] <- .a     |........|

+--------+--+--------------+---------------+----------------+--------+

|00010010|12|ld (de),a     |ld [.de],.a    |[.de] <- .a     |........|

+--------+--+--------------+---------------+----------------+--------+

|00110110|36|ld (hl),n     |ld [.hl],n     |[.hl] <- n      |........|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|01110 r |  |ld (hl),r     |ld [.hl],r     |[.hl] <- r      |........|

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ld (ii+d),n   |ld [ii+d],n    |[ii+d] <- n     |........|

|00110110|36|              |               |                |        |

|   d    |d |              |               |                |        |

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ld (ii+d),r   |ld [ii+d],r    |[ii+d] <- r     |........|

|01110 r |  |              |               |                |        |

|   d    |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00111010|3A|ld a,(nn)     |ld .a,[nn]     |.a <- [nn]      |........|

|  nnl   |  |              |               |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00001010|0A|ld a,(bc)     |ld .a,[.bc]    |.a <- [.bc]     |........|

+--------+--+--------------+---------------+----------------+--------+

|00011010|1A|ld a,(de)     |ld .a,[.de]    |.a <- [.de]     |........|

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ld a,i        |ld .a,.i       |.a <- .i        |**?0I?0*|

|01010111|57|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ld a,r        |ld .a,.r       |.a <- .r        |**?0I?0*|

|01011111|5F|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|01 r 110|  |ld r,(hl)     |ld r,[.hl]     |r <- [.hl]      |........|

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ld r,(ii+d)   |ld r,[ii+d]    |r <- [ii+d]     |........|

|01 r 110|  |              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00 r 110|  |ld r,n        |ld r,n         |r <- n          |........|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |ld u,n         |u <- n          |........|

|00 u 110|  |              |               |                |        |

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |ld v,n         |v <- n          |........|

|00 v 110|  |              |               |                |        |

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ld i,a        |ld .i,.a       |.i <- .a        |........|

|01000111|47|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|01 r1 r2|  |ld r1,r2      |ld r1,r2       |r1 <- r2        |........|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |ld u1,u2       |u1 <- u2        |........|

|01 u1 u2|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |ld v1,v2       |v1 <- v2        |........|

|01 v1 v2|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ld ss,(nn)    |ld ss,[nn]     |ssh <- [nn+1]   |........|

|01ss1011|  |              |               |ssl <- [nn]     |        |

|  nnl   |  |              |               |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00ss0001|  |ld ss,nn      |ld ss,nn       |ss <- nn        |........|

|  nnl   |  |              |               |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00101010|2A|ld hl,(nn)    |ld .hl,[nn]    |.h <- [nn+1]    |........|

|  nnl   |  |              |               |.l <- [nn]      |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ld ii,(nn)    |ld ii,[nn]     |iih <- [nn+1]   |........|

|00101010|2A|              |               |iil <- [nn]     |        |

|  nnl   |  |              |               |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ld ii,nn      |ld ii,nn       |ii <- nn        |........|

|00100001|21|              |               |                |        |

|  nnl   |  |              |               |                |        |

|  nnh   |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ld r,a        |ld .r,.a       |.r <- .a        |........|

|01001111|4F|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111001|F9|ld sp,hl      |ld .sp,.hl     |.sp <- .hl      |........|

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|ld sp,ii      |ld .sp,ii      |.sp <- ii       |........|

|11111001|F9|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ldd           |move           |[.de] <- [.hl]  |..?0$?0.|

|10101000|A8|(LoaD and     |[.hl--],[.de--]|.de <- .de - 1  |        |

|        |  |    Decrement)|(MOVE)         |.hl <- .hl - 1  |        |

|        |  |              |               |.bc <- .bc - 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|lddr          |movem          |repeat          |..?00?0.|

|10111000|B8|(LoaD,        |[.hl--],[.de--]|  [.de] <- [.hl]|        |

|        |  |  Decrement   |(MOVE Multiple)|  .de <- .de - 1|        |

|        |  |   and Repeat)|               |  .hl <- .hl - 1|        |

|        |  |              |               |  .bc <- .bc - 1|        |

|        |  |              |               |until .bc = 0   |        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ldi           |move           |[.de] <- [.hl]  |..?0$?0.|

|10100000|A0|(LoaD and     |[.hl++],[.de++]|.de <- .de + 1  |        |

|        |  |    Increment)|               |.hl <- .hl + 1  |        |

|        |  |              |               |.bc <- .bc - 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|ldir          |movem          |repeat          |..?00?0.|

|10110000|B0|(LoaD,        |[.hl++],[.de++]|  [.de] <- [.hl]|        |

|        |  |  Increment   |               |  .de <- .de + 1|        |

|        |  |   and Repeat)|               |  .hl <- .hl + 1|        |

|        |  |              |               |  .bc <- .bc - 1|        |

|        |  |              |               |until .bc = 0   |        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|              |mulub .a,r     |.hl <- .a * r   |0*?.0?.*|

|11 r 001|  |              |(MULtiply      |* note: using   |        |

|        |  |              | Unsigned Byte)|registers .a, .h|        |

|        |  |              |               |or .l for r may |        |

|        |  |              |               |generate strange|        |

|        |  |              |               |results         |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|              |muluw .hl,ss   |.de:.hl<-.hl*ss |0*?.0?.*|

|11ss0011|  |              |(MULtiply      |                |        |

|        |  |              | Unsigned Word)|* note: using   |        |

|        |  |              |               |registers .de or|        |

|        |  |              |               |.hl for ss may  |        |

|        |  |              |               |generate strange|        |

|        |  |              |               |results         |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|neg           |neg .a         |.a <- (~.a) + 1 |**?*o?1*|

|01000100|44|(NEGative)    |(NEGative)     |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00000000|00|nop           |nop            |no operation    |........|

|        |  |(No OPeration)|(No Operation) |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10110110|B6|or (hl)       |or .a,[.hl]    |.a <- .a # [.hl]|**?1p?00|

|        |  |(OR)          |(OR)           |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|or (ii+d)     |or .a,[ii+d]   |.a<-.a # [ii+d] |**?1p?00|

|10110110|B6|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11110110|F6|or n          |or .a,n        |.a <- .a # n    |**?1p?00|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10110 r |  |or r          |or .a,r        |.a <- .a # r    |**?1p?00|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |or .a,p        |.a <- .a # p    |**?1p?00|

|10110 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |or .a,q        |.a <- .a # q    |**?1p?00|

|10110 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|otdr          |outm           |repeat          |?1????1.|

|10111011|BB|(OuTput,      |   [.c],[.hl--]|  [.c] <- [.hl] |        |

|        |  |Decrement and |(OUTput        |  .b <- .b - 1  |        |

|        |  |       Repeat)|      Multiple)|  .hl <- .hl - 1|        |

|        |  |              |               |until .b = 0    |        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|otir          |outm           |repeat          |?1????1.|

|10110011|B3|(OuTput,      |   [.c],[.hl++]|  [.c] <- [.hl] |        |

|        |  |Increment and |(OUTput        |  .b <- .b - 1  |        |

|        |  |       Repeat)|      Multiple)|  .hl <- .hl + 1|        |

|        |  |              |               |until .b = 0    |        |

|        |  |              |               |* note: the 1st |        |

|        |  |              |               |execution time  |        |

|        |  |              |               |is for each loop|        |

|        |  |              |               |and the 2nd to  |        |

|        |  |              |               |the final loop  |        |

+--------+--+--------------+---------------+----------------+--------+

|11010011|D3|out (n),a     |out [n],.a     |[n] <- .a       |........|

|   n    |n |(OUTput)      |(OUTput)       |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|out (c),r     |out [.c],r     |[.c] <- .a      |........|

|01 r 001|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|outd          |out            |[.c] <- [.hl]   |?!????1.|

|10101011|AB|(OUTput and   |   [.c],[.hl--]|.b <- .b - 1    |        |

|        |  |    Decrement)|(OUTput)       |.hl <- .hl - 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|outi          |out            |[.c] <- [.hl]   |?!????1.|

|10100011|A3|(OUTput and   |   [.c],[.hl++]|.b <- .b - 1    |        |

|        |  |    Increment)|(OUTput)       |.hl <- .hl + 1  |        |

+--------+--+--------------+---------------+----------------+--------+

|11rr0001|  |pop rr        |pop rr         |rrl <- [.sp]    |........|

|        |  |(POP)         |(POP)          |rrh <- [.sp+1]  |        |

|        |  |              |               |.sp <- .sp + 2  |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|pop ii        |pop ii         |iil <- [.sp]    |........|

|11100001|E1|              |               |iih <- [.sp+1]  |        |

|        |  |              |               |.sp <- .sp + 2  |        |

+--------+--+--------------+---------------+----------------+--------+

|11rr0101|  |push rr       |push rr        |[.sp-2] <- rrl  |........|

|        |  |(PUSH)        |(PUSH)         |[.sp-1] <- rrh  |        |

|        |  |              |               |.sp <- .sp - 2  |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|push ii       |push ii        |[.sp-2] <- iil  |........|

|11100101|E5|              |               |[.sp-1] <- iih  |        |

|        |  |              |               |.sp <- .sp - 2  |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|res b,(hl)    |clr b,[.hl]    |Z <- ~ [.hl]{b} |?*?1??0.|

|10 b 110|  |(RESet bit)   |(CLeaR bit)    |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|res b,(ii+d)  |clr b,[ii+d]   |Z <- ~ [ii+d]{b}|?*?1??0.|

|11001011|CB|              |               |                |        |

|   d    |d |              |               |                |        |

|10 b 110|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|res b,r       |clr b,r        |Z <- ~ r{b}     |?*?1??0.|

|10 b  r |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001001|C9|ret           |ret            |.pcl <- [.sp]   |........|

|        |  |(RETurn)      |(RETurn)       |.pch <- [.sp+1] |        |

|        |  |              |               |.sp <- .sp + 2  |        |

+--------+--+--------------+---------------+----------------+--------+

|11011000|D8|ret c         |ret c          |if C = 1        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |        Carry)|         Carry)| .pch <- [.sp+1]|        |

|        |  |              |               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11111000|F8|ret m         |ret m          |if S = 1        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |        Minus)|         Minus)| .pch <- [.sp+1]|        |

|        |  |              |               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11010000|D0|ret nc        |ret nc         |if C = 0        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |    Not Carry)|     Not Carry)| .pch <- [.sp+1]|        |

|        |  |              |               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11000000|C0|ret nz        |ret nz         |if Z = 0        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |     Not Zero)|      Not Zero)| .pch <- [.sp+1]|        |

|        |  |              |               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11110000|F0|ret p         |ret p          |if S = 0        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |         Plus)|          Plus)| .pch <- [.sp+1]|        |

|        |  |              |               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11101000|E8|ret pe        |ret pe         |if P = 1        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |  Parity is   |Parity is Even)| .pch <- [.sp+1]|        |

|        |  |         Even)|               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11100000|E0|ret po        |ret po         |if P = 0        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |Parity is Odd)| Parity is Odd)| .pch <- [.sp+1]|        |

|        |  |              |               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11001000|C8|ret z         |ret z          |if Z = 1        |........|

|        |  |(RETurn if    |(RETurn if     | .pcl <- [.sp]  |........|

|        |  |         Zero)|          Zero)| .pch <- [.sp+1]|        |

|        |  |              |               | .sp <- .sp + 2 |        |

|        |  |              |               |else            |        |

|        |  |              |               |    no operation|        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|reti          |reti           |?               |........|

|01001101|4D|(RETurn from  |(RETurn from   |                |        |

|        |  | Interruption)|  Interruption)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|retn          |retn           |?               |........|

|01000101|45|(RETurn from  |(RETurn from   |                |        |

|        |  | Non-maskable | Non-maskable  |                |        |

|        |  | interruption)|  interruption)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rl (hl)       |rolc [.hl]     |tmp <- C        |**?0p?0*|

|00010110|16|(Rotate Left  |(ROtate Left   |C <- [.hl]{7}   |        |

|        |  |through carry)| through Carry)|[.hl] <- [.hl]*2|        |

|        |  |              |               |[.hl]{0} <- tmp |        |

|        |  |              |               |                |        |

|        |  |              |               |+--> Carry >---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-<[|||||||]<--+|        |

|        |  |              |               |  {7}[.hl]{0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|rl (ii+d)     |rolc [ii+d]    |tmp <- C        |**?0p?0*|

|11001011|CB|              |               |C <- [ii+d]{7}  |        |

|   d    |d |              |               |[ii+d]<-[ii+d]*2|        |

|00010110|16|              |               |[ii+d]{0} <- tmp|        |

|        |  |              |               |                |        |

|        |  |              |               |+--> Carry >---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-<[|||||||]<--+|        |

|        |  |              |               |  {7}[ii+d]{0}  |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rl r          |rolc r         |tmp <- C        |**?0p?0*|

|00010 r |  |              |               |C <- r{7}       |        |

|        |  |              |               |r <- r * 2      |        |

|        |  |              |               |r{0} <- tmp     |        |

|        |  |              |               |                |        |

|        |  |              |               |+--> Carry >---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-<[|||||||]<--+|        |

|        |  |              |               |  {7}  r  {0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|00010111|17|rla           |rolca          |tmp <- C        |..?0.?0*|

|        |  |(Rotate Left  |(ROtate Left   |C <- .a{7}      |        |

|        |  |through carry | through Carry |.a <- .a * 2    |        |

|        |  |  Accumulator)|   Accumulator)|.a{0} <- tmp    |        |

|        |  |              |               |                |        |

|        |  |              |               |+--> Carry >---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-<[|||||||]<--+|        |

|        |  |              |               |  {7} .a  {0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rlc (hl)      |rol [.hl]      |C <- [.hl]{7}   |**?0p?0*|

|00000110|06|(Rotate Left  |(ROtate Left)  |[.hl] <- [.hl]*2|        |

|        |  | with branch  |               |[.hl]{0} <- C   |        |

|        |  |        Carry)|               |                |        |

|        |  |              |               |+--> Carry      |        |

|        |  |              |               ||               |        |

|        |  |              |               |+------>-------+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-<[|||||||]<--+|        |

|        |  |              |               |  {7}[.hl]{0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|rlc (ii+d)    |rol [ii+d]     |C <- [ii+d]{7}  |**?0p?0*|

|11001011|CB|              |               |[ii+d]<-[ii+d]*2|        |

|   d    |d |              |               |[ii+d]{0} <- C  |        |

|00000110|06|              |               |                |        |

|        |  |              |               |+--> Carry      |        |

|        |  |              |               ||               |        |

|        |  |              |               |+------>-------+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-<[|||||||]<--+|        |

|        |  |              |               |  {7}[ii+d]{0}  |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rlc r         |rol r          |C <- r{7}       |**?0p?0*|

|00000 r |  |              |               |r <- r * 2      |        |

|        |  |              |               |r{0} <- C       |        |

|        |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00000111|07|rlca          |rola           |C <- .a{7}      |..?0.?0*|

|        |  |(Rotate Left  |(ROtate Left   |.a <- .a * 2    |        |

|        |  | with branch  |   Accumulator)|.a{0} <- C      |        |

|        |  |     Carry    |               |                |        |

|        |  |  Accumulator)|               |+--> Carry      |        |

|        |  |              |               ||               |        |

|        |  |              |               |+------>-------+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-<[|||||||]<--+|        |

|        |  |              |               |  {7} .a  {0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|rld           |rol4 [.hl]     |tmp <- .a{0..3} |..?0.?0*|

|01101111|6F|(Rotate Left  |(ROtate Left   |.a{0..3} <-     |        |

|        |  |      Decimal)|        4 bits)|     [.hl]{4..7}|        |

|        |  |              |               |[.hl]{4..7} <-  |        |

|        |  |              |               |     [.hl]{0..3}|        |

|        |  |              |               |[.hl]{0..3}<-tmp|        |

|        |  |              |               |                |        |

|        |  |              |               |[7..4|3..0]>---+|        |

|        |  |              |               |    .a  ^      ||        |

|        |  |              |               |        |      v|        |

|        |  |              |               |+--->---+      ||        |

|        |  |              |               ||              ||        |

|        |  |              |               |+-[7..4|3..0]<-+|        |

|        |  |              |               |     [.hl]      |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rr (hl)       |rorc [.hl]     |tmp <- C        |**?0p?0*|

|00011110|1E|(Rotate Right |(ROtate Right  |C <- [.hl]{0}   |        |

|        |  |through carry)| through Carry)|[.hl] <- [.hl]/2|        |

|        |  |              |               |[.hl]{7} <- tmp |        |

|        |  |              |               |                |        |

|        |  |              |               |+--< Carry <---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7}[.hl]{0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|rr (ii+d)     |rorc [ii+d]    |tmp <- C        |**?0p?0*|

|11001011|CB|              |               |C <- [ii+d]{0}  |        |

|   d    |d |              |               |[ii+d]<-[ii+d]/2|        |

|00011110|1E|              |               |[ii+d]{7} <- tmp|        |

|        |  |              |               |                |        |

|        |  |              |               |+--< Carry <---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7}[ii+d]{0}  |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rr r          |rorc r         |tmp <- C        |**?0p?0*|

|00011 r |  |              |               |C <- r{0}       |        |

|        |  |              |               |r <- r / 2      |        |

|        |  |              |               |r{7} <- tmp     |        |

|        |  |              |               |                |        |

|        |  |              |               |+--< Carry <---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7}  r  {0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|00011111|1F|rra           |rorca          |tmp <- C        |..?0.?0*|

|        |  |(Rotate Right |(ROtate Right  |C <- .a{7}      |        |

|        |  |through carry | through Carry |.a <- .a * 2    |        |

|        |  |  Accumulator)|   Accumulator)|.a{0} <- tmp    |        |

|        |  |              |               |                |        |

|        |  |              |               |+--< Carry <---+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7} .a  {0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rrc (hl)      |ror [.hl]      |C <- [.hl]{0}   |**?0p?0*|

|00001110|0E|(Rotate Right |(ROtate Right) |[.hl] <- [.hl]/2|        |

|        |  | with branch  |               |[.hl]{7} <- C   |        |

|        |  |        Carry)|               |                |        |

|        |  |              |               |     Carry <---+|        |

|        |  |              |               |               ||        |

|        |  |              |               |+------<-------+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7}[.hl]{0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|rrc (ii+d)    |ror [ii+d]     |C <- [ii+d]{0}  |**?0p?0*|

|11001011|CB|              |               |[ii+d]<-[ii+d]/2|        |

|   d    |d |              |               |[ii+d]{7} <- C  |        |

|00001110|0E|              |               |                |        |

|        |  |              |               |     Carry <---+|        |

|        |  |              |               |               ||        |

|        |  |              |               |+------<-------+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7}[ii+d]{0}  |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|rrc r         |ror r          |C <- r{0}       |**?0p?0*|

|00001 r |  |              |               |r <- r / 2      |        |

|        |  |              |               |r{7} <- C       |        |

|        |  |              |               |                |        |

|        |  |              |               |     Carry <---+|        |

|        |  |              |               |               ||        |

|        |  |              |               |+------<-------+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7}  r  {0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|00001111|0F|rrca          |rora           |C <- .a{0}      |..?0.?0*|

|        |  |(Rotate Right |(ROtate Right  |.a <- .a / 2    |        |

|        |  | with branch  |   Accumulator)|.a{7} <- C      |        |

|        |  |     Carry    |               |                |        |

|        |  |  Accumulator)|               |     Carry <---+|        |

|        |  |              |               |               ||        |

|        |  |              |               |+------<-------+|        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[|||||||]>--+|        |

|        |  |              |               |  {7} .a  {0}   |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|rrd           |rol4 [.hl]     |tmp <- .a{0..3} |..?0.?0*|

|01100111|67|(Rotate Right |(ROtate Right  |.a{0..3} <-     |        |

|        |  |      Decimal)|        4 bits)|     [.hl]{0..3}|        |

|        |  |              |               |[.hl]{0..3} <-  |        |

|        |  |              |               |     [.hl]{4..7}|        |

|        |  |              |               |[.hl]{4..7}<-tmp|        |

|        |  |              |               |                |        |

|        |  |              |               |[7..4|3..0]<---+|        |

|        |  |              |               |    .a  v      ||        |

|        |  |              |               |        |      ^|        |

|        |  |              |               |+---<---+      ||        |

|        |  |              |               ||              ||        |

|        |  |              |               |+->[7..4|3..0]-+|        |

|        |  |              |               |     [.hl]      |        |

+--------+--+--------------+---------------+----------------+--------+

|11 k 111|  |rst k         |brk k          |[.sp-2] <- .pcl |........|

|        |  |(ReSeT)       |(BReaK)        |[.sp-1] <- .pch |        |

|        |  |              |               |.sp <- .sp - 2  |        |

|        |  |              |               |.pcl <- k       |        |

|        |  |              |               |.pch <- 0       |        |

+--------+--+--------------+---------------+----------------+--------+

|10011110|9E|sbc a,(hl)    |subc .a,[.hl]  |.a <- .a-[.hl]-C|**?*o?0*|

|        |  |(SuBtract with|(SUBtract with |                |        |

|        |  |        Carry)|         Carry)|                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|sbc a,(ii+d)  |subc .a,[ii+d] |.a<-.a-[ii+d]-C |**?*o?0*|

|10011110|9E|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11011110|DE|sbc a,n       |subc .a,n      |.a <- .a - n - C|**?*o?0*|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10011 r |  |sbc a,r       |subc .a,r      |.a <- .a - r - C|**?*o?0*|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |subc .a,p      |.a <- .a - p - C|**?*o?0*|

|10011 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |subc .a,q      |.a <- .a - q - C|**?*o?0*|

|10011 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101101|ED|sbc hl,ss     |subc .hl,ss    |.hl <- .hl-ss-C |**?*o?0*|

|01ss0010|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|00110111|37|scf           |setc           |C <- 1          |..?0.?01|

|        |  |(Set Carry    |(SET Carry)    |                |        |

|        |  |         Flag)|               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|set b,(hl)    |set b,[.hl]    |Z <- ~ [.hl]{b} |?*?1??0.|

|11 b 110|  |(SET bit)     |(SET bit)      |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|set b,(ii+d)  |set b,[ii+d]   |Z <- ~ [ii+d]{b}|?*?1??0.|

|11001011|CB|              |               |                |        |

|   d    |d |              |               |                |        |

|11 b 110|  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|set b,r       |set b,r        |Z <- ~ r{b}     |?*?1??0.|

|11 b  r |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|sla (hl)      |shl [.hl]      |C <- [.hl]{7}   |**?*o?0*|

|00100110|26|(Shift Left   |shla [.hl]     |[.hl] <- [.hl]*2|        |

|        |  | And clear    |(SHift Left    |                |        |

|        |  |    least     | [And clear    |                |        |

|        |  |  significant |     least     |                |        |

|        |  |          bit)|   significant |                |        |

|        |  |              |          bit])|                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|sla (ii+d)    |shl [ii+d]     |C <- [ii+d]{7}  |**?*o?0*|

|11001011|CB|              |shla [ii+d]    |[ii+d]<-[ii+d]*2|        |

|   d    |d |              |               |                |        |

|00100110|26|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|sla r         |shl r          |C <- r{7}       |**?*o?0*|

|00100 r |  |              |shla r         |r <- r * 2      |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|sra (hl)      |shra [.hl]     |tmp <- [.hl]{7} |**?*o?0*|

|00101110|2E|(Shift Right  |(SHift Right   |C <- [.hl]{0}   |        |

|        |  | And preserv  | And preserv   |[.hl] <- [.hl]/2|        |

|        |  |     most     |     most      |[.hl]{7} <- tmp |        |

|        |  |  significant |   significant |                |        |

|        |  |         bit) |          bit])|                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|sra (ii+d)    |shra [ii+d]    |tmp <- [ii+d]{7}|**?*o?0*|

|11001011|CB|              |               |C <- [ii+d]{0}  |        |

|   d    |d |              |               |[ii+d]<-[ii+d]*2|        |

|00101110|2E|              |               |[ii+d]{7} <- tmp|        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|sra r         |shra r         |tmp <- r{7}     |**?*o?0*|

|00101 r |  |              |               |C <- r{0}       |        |

|        |  |              |               |r <- r / 2      |        |

|        |  |              |               |r{7} <- tmp     |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|srl (hl)      |shr [.hl]      |C <- [.hl]{0}   |**?*o?0*|

|00111110|3E|(Shift Left   |(SHift Right)  |[.hl] <- [.hl]/2|        |

|        |  |and cLear most|               |                |        |

|        |  |  significant |               |                |        |

|        |  |          bit)|               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|srl (ii+d)    |shr [ii+d]     |C <- [ii+d]{0}  |**?*o?0*|

|11001011|CB|              |               |[ii+d]<-[ii+d]/2|        |

|   d    |d |              |               |                |        |

|00111110|3E|              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11001011|CB|srl r         |shr r          |C <- r{0}       |**?*o?0*|

|00111 r |  |              |               |r <- r / 2      |        |

+--------+--+--------------+---------------+----------------+--------+

|10010110|96|sub (hl)      |sub .a,[.hl]   |.a <- .a - [.hl]|**?*o?0*|

|        |  |(SUBtract)    |(SUBtract)     |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|sub (ii+d)    |sub .a,[ii+d]  |.a <- .a-[ii+d] |**?*o?0*|

|10010110|96|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11010110|D6|sub n         |sub .a,n       |.a <- .a - n    |**?*o?0*|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10010 r |  |sub r         |sub .a,r       |.a <- .a - r    |**?*o?0*|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |sub .a,p       |.a <- .a - p    |**?*o?0*|

|10010 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |sub .a,q       |.a <- .a - q    |**?*o?0*|

|10010 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10101110|AE|xor (hl)      |xor .a,[.hl]   |.a <- .a ^ [.hl]|**?1p?00|

|        |  |(eXclusive OR)|(eXclusive OR) |                |        |

+--------+--+--------------+---------------+----------------+--------+

|   ii   |ii|xor (ii+d)    |xor .a,[ii+d]  |.a <- .a^[ii+d] |**?1p?00|

|10101110|AE|              |               |                |        |

|   d    |d |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11101110|EE|xor n         |xor .a,n       |.a <- .a ^ n    |**?1p?00|

|   n    |n |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|10101 r |  |xor r         |xor .a,r       |.a <- .a ^ r    |**?1p?00|

+--------+--+--------------+---------------+----------------+--------+

|11011101|DD|              |xor .a,p       |.a <- .a ^ p    |**?1p?00|

|10101 p |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+

|11111101|FD|              |xor .a,q       |.a <- .a ^ q    |**?1p?00|

|10101 q |  |              |               |                |        |

+--------+--+--------------+---------------+----------------+--------+