Z80 and R800 Assembly/Machine Language Op-code Tables

Written by "Cyberknight" Masao Kawata.

E-Mail: cyber_unicorn@hotmail.com

Revised: 1997-10-06 Monday.

Notes:

*1: the op-code "ED 70" has no mnemonic defined for Z80,

though it is a valid instruction.

*2: SHL and SHLA instructions of the R800 are the same,

but with two different mnemonics defined, so they

can "pair" the SHR and SHRA instruction mnemonics.

*3: some op-codes are equivalent, performing the same

operations, so they are assigned to the same

mnemonics.

+-------------------------------------------------------- +

|            Fast Reference (sorted op-codes)             |

+-------------+-----------------+-------------------------+

|  Op-codes   |   Z80 Assembly  |     R800 Assembly       |

|(Hexadecimal)|    Mnemonics    |       Mnemonics         |

+=============+=================+=========================+

|00           |nop              |nop                      |

|01 nnl nnh   |ld bc,nn         |ld .bc,nn                |

|02           |ld (bc),a        |ld [.bc],.a              |

|03           |inc bc           |inc .bc                  |

|04           |inc b            |inc .b                   |

|05           |dec b            |dec .b                   |

|06 n         |ld b,n           |ld .b,n                  |

|07           |rlca             |rola                     |

|08           |ex af,af'        |xch .af,.af'             |

|09           |add hl,bc        |add .hl,.bc              |

|0A           |ld a,(bc)        |ld .a,[.bc]              |

|0B           |dec bc           |dec .bc                  |

|0C           |inc c            |inc .c                   |

|0D           |dec c            |dec .c                   |

|0E n         |ld c,n           |ld .c,n                  |

|0F           |rrca             |rora                     |

+-------------+-----------------+-------------------------+

|10 d         |djnz d           |dbnz d                   |

|11 nnl nnh   |ld de,nn         |ld .de,nn                |

|12           |ld (de),a        |ld [.de],.a              |

|13           |inc de           |inc .de                  |

|14           |inc d            |inc .d                   |

|15           |dec d            |dec .d                   |

|16 n         |ld d,n           |ld .d,n                  |

|17           |rla              |rolca                    |

|18 d         |jr d             |short br d               |

|19           |add hl,de        |add .hl,.de              |

|1A           |ld a,(de)        |ld .a,[.de]              |

|1B           |dec de           |dec .de                  |

|1C           |inc e            |inc .e                   |

|1D           |dec e            |dec .e                   |

|1E n         |ld e,n           |ld .e,n                  |

|1F           |rra              |rorca                    |

+-------------+-----------------+-------------------------+

|20 d         |jr nz,d          |short bnz d              |

|21 nnl nnh   |ld hl,nn         |ld .hl,nn                |

|22 nnl nnh   |ld (nn),hl       |ld [nn],.hl              |

|23           |inc hl           |inc .hl                  |

|24           |inc h            |inc .h                   |

|25           |dec h            |dec .h                   |

|26 n         |ld h,n           |ld .h,n                  |

|27           |daa              |adj .a                   |

|28 d         |jr z,d           |short bz d               |

|29           |add hl,hl        |add .hl,.hl              |

|2A nnl nnh   |ld hl,(nn)       |ld .hl,[nn]              |

|2B           |dec hl           |dec .hl                  |

|2C           |inc l            |inc .l                   |

|2D           |dec l            |dec .l                   |

|2E n         |ld l,n           |ld .l,n                  |

|2F           |cpl              |not .a                   |

+-------------+-----------------+-------------------------+

|30 d         |jr nc,d          |short bnc d              |

|31 nnl nnh   |ld sp,nn         |ld .sp,nn                |

|32 nnl nnh   |ld (nn),a        |ld [nn],.a               |

|33           |inc sp           |inc .sp                  |

|34           |inc (hl)         |inc [.hl]                |

|35           |dec (hl)         |dec [.hl]                |

|36 n         |ld (hl),n        |ld [.hl],n               |

|37           |scf              |setc                     |

|38 d         |jr c,d           |short bc d               |

|39           |add hl,sp        |add .hl,.sp              |

|3A nnl nnh   |ld a,(nn)        |ld .a,[nn]               |

|3B           |dec sp           |dec .sp                  |

|3C           |inc a            |inc .a                   |

|3D           |dec a            |dec .a                   |

|3E n         |ld a,n           |ld .a,n                  |

|3F           |ccf              |notc                     |

+-------------+-----------------+-------------------------+

|40           |ld b,b           |ld .b,.b                 |

|41           |ld b,c           |ld .b,.c                 |

|42           |ld b,d           |ld .b,.d                 |

|43           |ld b,e           |ld .b,.e                 |

|44           |ld b,h           |ld .b,.h                 |

|45           |ld b,l           |ld .b,.l                 |

|46           |ld b,(hl)        |ld .b,[.hl]              |

|47           |ld b,a           |ld .b,.a                 |

|48           |ld c,b           |ld .c,.b                 |

|49           |ld c,c           |ld .c,.c                 |

|4A           |ld c,d           |ld .c,.d                 |

|4B           |ld c,e           |ld .c,.e                 |

|4C           |ld c,h           |ld .c,.h                 |

|4D           |ld c,l           |ld .c,.l                 |

|4E           |ld c,(hl)        |ld .c,[.hl]              |

|4F           |ld c,a           |ld .c,.a                 |

+-------------+-----------------+-------------------------+

|50           |ld d,b           |ld .d,.b                 |

|51           |ld d,c           |ld .d,.c                 |

|52           |ld d,d           |ld .d,.d                 |

|53           |ld d,e           |ld .d,.e                 |

|54           |ld d,h           |ld .d,.h                 |

|55           |ld d,l           |ld .d,.l                 |

|56           |ld d,(hl)        |ld .d,[.hl]              |

|57           |ld d,a           |ld .d,.a                 |

|58           |ld e,b           |ld .e,.b                 |

|59           |ld e,c           |ld .e,.c                 |

|5A           |ld e,d           |ld .e,.d                 |

|5B           |ld e,e           |ld .e,.e                 |

|5C           |ld e,h           |ld .e,.h                 |

|5D           |ld e,l           |ld .e,.l                 |

|5E           |ld e,(hl)        |ld .e,[.hl]              |

|5F           |ld e,a           |ld .e,.a                 |

+-------------+-----------------+-------------------------+

|60           |ld h,b           |ld .h,.b                 |

|61           |ld h,c           |ld .h,.c                 |

|62           |ld h,d           |ld .h,.d                 |

|63           |ld h,e           |ld .h,.e                 |

|64           |ld h,h           |ld .h,.h                 |

|65           |ld h,l           |ld .h,.l                 |

|66           |ld h,(hl)        |ld .h,[.hl]              |

|67           |ld h,a           |ld .h,.a                 |

|68           |ld l,b           |ld .l,.b                 |

|69           |ld l,c           |ld .l,.c                 |

|6A           |ld l,d           |ld .l,.d                 |

|6B           |ld l,e           |ld .l,.e                 |

|6C           |ld l,h           |ld .l,.h                 |

|6D           |ld l,l           |ld .l,.l                 |

|6E           |ld l,(hl)        |ld .l,[.hl]              |

|6F           |ld l,a           |ld .l,.a                 |

+-------------+-----------------+-------------------------+

|70           |ld (hl),b        |ld [.hl],.b              |

|71           |ld (hl),c        |ld [.hl],.c              |

|72           |ld (hl),d        |ld [.hl],.d              |

|73           |ld (hl),e        |ld [.hl],.e              |

|74           |ld (hl),h        |ld [.hl],.h              |

|75           |ld (hl),l        |ld [.hl],.l              |

|76           |halt             |halt                     |

|77           |ld (hl),a        |ld [.hl],.a              |

|78           |ld a,b           |ld .a,.b                 |

|79           |ld a,c           |ld .a,.c                 |

|7A           |ld a,d           |ld .a,.d                 |

|7B           |ld a,e           |ld .a,.e                 |

|7C           |ld a,h           |ld .a,.h                 |

|7D           |ld a,l           |ld .a,.l                 |

|7E           |ld a,(hl)        |ld .a,[.hl]              |

|7F           |ld a,a           |ld .a,.a                 |

+-------------+-----------------+-------------------------+

|80           |add a,b          |add .a,.b                |

|81           |add a,c          |add .a,.c                |

|82           |add a,d          |add .a,.d                |

|83           |add a,e          |add .a,.e                |

|84           |add a,h          |add .a,.h                |

|85           |add a,l          |add .a,.l                |

|86           |add a,(hl)       |add .a,[.hl]             |

|87           |add a,a          |add .a,.a                |

|88           |adc a,b          |addc .a,.b               |

|89           |adc a,c          |addc .a,.c               |

|8A           |adc a,d          |addc .a,.d               |

|8B           |adc a,e          |addc .a,.e               |

|8C           |adc a,h          |addc .a,.h               |

|8D           |adc a,l          |addc .a,.l               |

|8E           |adc a,(hl)       |addc .a,[.hl]            |

|8F           |adc a,a          |addc .a,.a               |

+-------------+-----------------+-------------------------+

|90           |sub b            |sub .a,.b                |

|91           |sub c            |sub .a,.c                |

|92           |sub d            |sub .a,.d                |

|93           |sub e            |sub .a,.e                |

|94           |sub h            |sub .a,.h                |

|95           |sub l            |sub .a,.l                |

|96           |sub (hl)         |sub .a,[.hl]             |

|97           |sub a            |sub .a,.a                |

|98           |sbc a,b          |subc .a,.b               |

|99           |sbc a,c          |subc .a,.c               |

|9A           |sbc a,d          |subc .a,.d               |

|9B           |sbc a,e          |subc .a,.e               |

|9C           |sbc a,h          |subc .a,.h               |

|9D           |sbc a,l          |subc .a,.l               |

|9E           |sbc a,(hl)       |subc .a,[.hl]            |

|9F           |sbc a,a          |subc .a,.a               |

+-------------+-----------------+-------------------------+

|A0           |and b            |and .a,.b                |

|A1           |and c            |and .a,.c                |

|A2           |and d            |and .a,.d                |

|A3           |and e            |and .a,.e                |

|A4           |and h            |and .a,.h                |

|A5           |and l            |and .a,.l                |

|A6           |and (hl)         |and .a,[.hl]             |

|A7           |and a            |and .a,.a                |

|A8           |xor b            |xor .a,.b                |

|A9           |xor c            |xor .a,.c                |

|AA           |xor d            |xor .a,.d                |

|AB           |xor e            |xor .a,.e                |

|AC           |xor h            |xor .a,.h                |

|AD           |xor l            |xor .a,.l                |

|AE           |xor (hl)         |xor .a,[.hl]             |

|AF           |xor a            |xor .a,.a                |

+-------------+-----------------+-------------------------+

|B0           |or b             |or .a,.b                 |

|B1           |or c             |or .a,.c                 |

|B2           |or d             |or .a,.d                 |

|B3           |or e             |or .a,.e                 |

|B4           |or h             |or .a,.h                 |

|B5           |or l             |or .a,.l                 |

|B6           |or (hl)          |or .a,[.hl]              |

|B7           |or a             |or .a,.a                 |

|B8           |cp b             |cmp .a,.b                |

|B9           |cp c             |cmp .a,.c                |

|BA           |cp d             |cmp .a,.d                |

|BB           |cp e             |cmp .a,.e                |

|BC           |cp h             |cmp .a,.h                |

|BD           |cp l             |cmp .a,.l                |

|BE           |cp (hl)          |cmp .a,[.hl]             |

|BF           |cp a             |cmp .a,.a                |

+-------------+-----------------+-------------------------+

|C0           |ret nz           |ret nz                   |

|C1           |pop bc           |pop .bc                  |

|C2 nnl nnh   |jp nz,nn         |bnz nn                   |

|C3 nnl nnh   |jp nn            |br nn                    |

|C4 nnl nnh   |call nz,nn       |call nz,nn               |

|C5           |push bc          |push .bc                 |

|C6 n         |add a,n          |add .a,n                 |

|C7           |rst 00H          |brk 00H                  |

|C8           |ret z            |ret z                    |

|C9           |ret              |ret                      |

|CA nnl nnh   |jp z,nn          |bz nn                    |

+=============+=================+=========================+

|CB 00        |rlc b            |rol .b                   |

|CB 01        |rlc c            |rol .c                   |

|CB 02        |rlc d            |rol .d                   |

|CB 03        |rlc e            |rol .e                   |

|CB 04        |rlc h            |rol .h                   |

|CB 05        |rlc l            |rol .l                   |

|CB 06        |rlc (hl)         |rol [.hl]                |

|CB 07        |rlc a            |rol .a                   |

|CB 08        |rrc b            |ror .b                   |

|CB 09        |rrc c            |ror .c                   |

|CB 0A        |rrc d            |ror .d                   |

|CB 0B        |rrc e            |ror .e                   |

|CB 0C        |rrc h            |ror .h                   |

|CB 0D        |rrc l            |ror .l                   |

|CB 0E        |rrc (hl)         |ror [.hl]                |

|CB 0F        |rrc a            |ror .a                   |

+-------------+-----------------+-------------------------+

|CB 10        |rl b             |rolc .b                  |

|CB 11        |rl c             |rolc .c                  |

|CB 12        |rl d             |rolc .d                  |

|CB 13        |rl e             |rolc .e                  |

|CB 14        |rl h             |rolc .h                  |

|CB 15        |rl l             |rolc .l                  |

|CB 16        |rl (hl)          |rolc [.hl]               |

|CB 17        |rl a             |rolc .a                  |

|CB 18        |rr b             |rorc .b                  |

|CB 19        |rr c             |rorc .c                  |

|CB 1A        |rr d             |rorc .d                  |

|CB 1B        |rr e             |rorc .e                  |

|CB 1C        |rr h             |rorc .h                  |

|CB 1D        |rr l             |rorc .l                  |

|CB 1E        |rr (hl)          |rorc [.hl]               |

|CB 1F        |rr a             |rorc .a                  |

+-------------+-----------------+------------+------------+

|CB 20        |sla a            |shl .b      |shla .b     |

|CB 21        |sla a            |shl .c      |shla .c     |

|CB 22        |sla a            |shl .d      |shla .d     |

|CB 23        |sla a            |shl .e      |shla .e     |

|CB 24        |sla a            |shl .h      |shla .h     |

|CB 25        |sla a            |shl .l      |shla .l     |

|CB 26        |sla (hl)         |shl [.hl]   |shla [.hl]  |

|CB 27        |sla a            |shl .a      |shla .a     |

+-------------+-----------------+------------+------------+

|CB 28        |sra b            |shra .b                  |

|CB 29        |sra c            |shra .c                  |

|CB 2A        |sra d            |shra .d                  |

|CB 2B        |sra e            |shra .e                  |

|CB 2C        |sra h            |shra .h                  |

|CB 2D        |sra l            |shra .l                  |

|CB 2E        |sra (hl)         |shra [.hl]               |

|CB 2F        |sra a            |shra .a                  |

+-------------+-----------------+-------------------------+

|CB 30        |                                           |

|.............|             invalid op-codes              |

|CB 37        |                                           |

+-------------+-----------------+-------------------------+

|CB 38        |srl b            |shr .b                   |

|CB 39        |srl c            |shr .c                   |

|CB 3A        |srl d            |shr .d                   |

|CB 3B        |srl e            |shr .e                   |

|CB 3C        |srl h            |shr .h                   |

|CB 3D        |srl l            |shr .l                   |

|CB 3E        |srl (hl)         |shr [.hl]                |

|CB 3F        |srl a            |shr .a                   |

+-------------+-----------------+-------------------------+

|CB 40        |bit 0,b          |bit 0,.b                 |

|CB 41        |bit 0,c          |bit 0,.c                 |

|CB 42        |bit 0,d          |bit 0,.d                 |

|CB 43        |bit 0,e          |bit 0,.e                 |

|CB 44        |bit 0,h          |bit 0,.h                 |

|CB 45        |bit 0,l          |bit 0,.l                 |

|CB 46        |bit 0,(hl)       |bit 0,[.hl]              |

|CB 47        |bit 0,a          |bit 0,.a                 |

|CB 48        |bit 1,b          |bit 1,.b                 |

|CB 49        |bit 1,c          |bit 1,.c                 |

|CB 4A        |bit 1,d          |bit 1,.d                 |

|CB 4B        |bit 1,e          |bit 1,.e                 |

|CB 4C        |bit 1,h          |bit 1,.h                 |

|CB 4D        |bit 1,l          |bit 1,.l                 |

|CB 4E        |bit 1,(hl)       |bit 1,[.hl]              |

|CB 4F        |bit 1,a          |bit 1,.a                 |

+-------------+-----------------+-------------------------+

|CB 50        |bit 2,b          |bit 2,.b                 |

|CB 51        |bit 2,c          |bit 2,.c                 |

|CB 52        |bit 2,d          |bit 2,.d                 |

|CB 53        |bit 2,e          |bit 2,.e                 |

|CB 54        |bit 2,h          |bit 2,.h                 |

|CB 55        |bit 2,l          |bit 2,.l                 |

|CB 56        |bit 2,(hl)       |bit 2,[.hl]              |

|CB 57        |bit 2,a          |bit 2,.a                 |

|CB 58        |bit 3,b          |bit 3,.b                 |

|CB 59        |bit 3,c          |bit 3,.c                 |

|CB 5A        |bit 3,d          |bit 3,.d                 |

|CB 5B        |bit 3,e          |bit 3,.e                 |

|CB 5C        |bit 3,h          |bit 3,.h                 |

|CB 5D        |bit 3,l          |bit 3,.l                 |

|CB 5E        |bit 3,(hl)       |bit 3,[.hl]              |

|CB 5F        |bit 3,a          |bit 3,.a                 |

+-------------+-----------------+-------------------------+

|CB 60        |bit 4,b          |bit 4,.b                 |

|CB 61        |bit 4,c          |bit 4,.c                 |

|CB 62        |bit 4,d          |bit 4,.d                 |

|CB 63        |bit 4,e          |bit 4,.e                 |

|CB 64        |bit 4,h          |bit 4,.h                 |

|CB 65        |bit 4,l          |bit 4,.l                 |

|CB 66        |bit 4,(hl)       |bit 4,[.hl]              |

|CB 67        |bit 4,a          |bit 4,.a                 |

|CB 68        |bit 5,b          |bit 5,.b                 |

|CB 69        |bit 5,c          |bit 5,.c                 |

|CB 6A        |bit 5,d          |bit 5,.d                 |

|CB 6B        |bit 5,e          |bit 5,.e                 |

|CB 6C        |bit 5,h          |bit 5,.h                 |

|CB 6D        |bit 5,l          |bit 5,.l                 |

|CB 6E        |bit 5,(hl)       |bit 5,[.hl]              |

|CB 6F        |bit 5,a          |bit 5,.a                 |

+-------------+-----------------+-------------------------+

|CB 70        |bit 6,b          |bit 6,.b                 |

|CB 71        |bit 6,c          |bit 6,.c                 |

|CB 72        |bit 6,d          |bit 6,.d                 |

|CB 73        |bit 6,e          |bit 6,.e                 |

|CB 74        |bit 6,h          |bit 6,.h                 |

|CB 75        |bit 6,l          |bit 6,.l                 |

|CB 76        |bit 6,(hl)       |bit 6,[.hl]              |

|CB 77        |bit 6,a          |bit 6,.a                 |

|CB 78        |bit 7,b          |bit 7,.b                 |

|CB 79        |bit 7,c          |bit 7,.c                 |

|CB 7A        |bit 7,d          |bit 7,.d                 |

|CB 7B        |bit 7,e          |bit 7,.e                 |

|CB 7C        |bit 7,h          |bit 7,.h                 |

|CB 7D        |bit 7,l          |bit 7,.l                 |

|CB 7E        |bit 7,(hl)       |bit 7,[.hl]              |

|CB 7F        |bit 7,a          |bit 7,.a                 |

+-------------+-----------------+-------------------------+

|CB 80        |res 0,b          |clr 0,.b                 |

|CB 81        |res 0,c          |clr 0,.c                 |

|CB 82        |res 0,d          |clr 0,.d                 |

|CB 83        |res 0,e          |clr 0,.e                 |

|CB 84        |res 0,h          |clr 0,.h                 |

|CB 85        |res 0,l          |clr 0,.l                 |

|CB 86        |res 0,(hl)       |clr 0,[.hl]              |

|CB 87        |res 0,a          |clr 0,.a                 |

|CB 88        |res 1,b          |clr 1,.b                 |

|CB 89        |res 1,c          |clr 1,.c                 |

|CB 8A        |res 1,d          |clr 1,.d                 |

|CB 8B        |res 1,e          |clr 1,.e                 |

|CB 8C        |res 1,h          |clr 1,.h                 |

|CB 8D        |res 1,l          |clr 1,.l                 |

|CB 8E        |res 1,(hl)       |clr 1,[.hl]              |

|CB 8F        |res 1,a          |clr 1,.a                 |

+-------------+-----------------+-------------------------+

|CB 90        |res 2,b          |clr 2,.b                 |

|CB 91        |res 2,c          |clr 2,.c                 |

|CB 92        |res 2,d          |clr 2,.d                 |

|CB 93        |res 2,e          |clr 2,.e                 |

|CB 94        |res 2,h          |clr 2,.h                 |

|CB 95        |res 2,l          |clr 2,.l                 |

|CB 96        |res 2,(hl)       |clr 2,[.hl]              |

|CB 97        |res 2,a          |clr 2,.a                 |

|CB 98        |res 3,b          |clr 3,.b                 |

|CB 99        |res 3,c          |clr 3,.c                 |

|CB 9A        |res 3,d          |clr 3,.d                 |

|CB 9B        |res 3,e          |clr 3,.e                 |

|CB 9C        |res 3,h          |clr 3,.h                 |

|CB 9D        |res 3,l          |clr 3,.l                 |

|CB 9E        |res 3,(hl)       |clr 3,[.hl]              |

|CB 9F        |res 3,a          |clr 3,.a                 |

+-------------+-----------------+-------------------------+

|CB A0        |res 4,b          |clr 4,.b                 |

|CB A1        |res 4,c          |clr 4,.c                 |

|CB A2        |res 4,d          |clr 4,.d                 |

|CB A3        |res 4,e          |clr 4,.e                 |

|CB A4        |res 4,h          |clr 4,.h                 |

|CB A5        |res 4,l          |clr 4,.l                 |

|CB A6        |res 4,(hl)       |clr 4,[.hl]              |

|CB A7        |res 4,a          |clr 4,.a                 |

|CB A8        |res 5,b          |clr 5,.b                 |

|CB A9        |res 5,c          |clr 5,.c                 |

|CB AA        |res 5,d          |clr 5,.d                 |

|CB AB        |res 5,e          |clr 5,.e                 |

|CB AC        |res 5,h          |clr 5,.h                 |

|CB AD        |res 5,l          |clr 5,.l                 |

|CB AE        |res 5,(hl)       |clr 5,[.hl]              |

|CB AF        |res 5,a          |clr 5,.a                 |

+-------------+-----------------+-------------------------+

|CB B0        |res 6,b          |clr 6,.b                 |

|CB B1        |res 6,c          |clr 6,.c                 |

|CB B2        |res 6,d          |clr 6,.d                 |

|CB B3        |res 6,e          |clr 6,.e                 |

|CB B4        |res 6,h          |clr 6,.h                 |

|CB B5        |res 6,l          |clr 6,.l                 |

|CB B6        |res 6,(hl)       |clr 6,[.hl]              |

|CB B7        |res 6,a          |clr 6,.a                 |

|CB B8        |res 7,b          |clr 7,.b                 |

|CB B9        |res 7,c          |clr 7,.c                 |

|CB BA        |res 7,d          |clr 7,.d                 |

|CB BB        |res 7,e          |clr 7,.e                 |

|CB BC        |res 7,h          |clr 7,.h                 |

|CB BD        |res 7,l          |clr 7,.l                 |

|CB BE        |res 7,(hl)       |clr 7,[.hl]              |

|CB BF        |res 7,a          |clr 7,.a                 |

+-------------+-----------------+-------------------------+

|CB C0        |set 0,b          |set 0,.b                 |

|CB C1        |set 0,c          |set 0,.c                 |

|CB C2        |set 0,d          |set 0,.d                 |

|CB C3        |set 0,e          |set 0,.e                 |

|CB C4        |set 0,h          |set 0,.h                 |

|CB C5        |set 0,l          |set 0,.l                 |

|CB C6        |set 0,(hl)       |set 0,[.hl]              |

|CB C7        |set 0,a          |set 0,.a                 |

|CB C8        |set 1,b          |set 1,.b                 |

|CB C9        |set 1,c          |set 1,.c                 |

|CB CA        |set 1,d          |set 1,.d                 |

|CB CB        |set 1,e          |set 1,.e                 |

|CB CC        |set 1,h          |set 1,.h                 |

|CB CD        |set 1,l          |set 1,.l                 |

|CB CE        |set 1,(hl)       |set 1,[.hl]              |

|CB CF        |set 1,a          |set 1,.a                 |

+-------------+-----------------+-------------------------+

|CB D0        |set 2,b          |set 2,.b                 |

|CB D1        |set 2,c          |set 2,.c                 |

|CB D2        |set 2,d          |set 2,.d                 |

|CB D3        |set 2,e          |set 2,.e                 |

|CB D4        |set 2,h          |set 2,.h                 |

|CB D5        |set 2,l          |set 2,.l                 |

|CB D6        |set 2,(hl)       |set 2,[.hl]              |

|CB D7        |set 2,a          |set 2,.a                 |

|CB D8        |set 3,b          |set 3,.b                 |

|CB D9        |set 3,c          |set 3,.c                 |

|CB DA        |set 3,d          |set 3,.d                 |

|CB DB        |set 3,e          |set 3,.e                 |

|CB DC        |set 3,h          |set 3,.h                 |

|CB DD        |set 3,l          |set 3,.l                 |

|CB DE        |set 3,(hl)       |set 3,[.hl]              |

|CB DF        |set 3,a          |set 3,.a                 |

+-------------+-----------------+-------------------------+

|CB E0        |set 4,b          |set 4,.b                 |

|CB E1        |set 4,c          |set 4,.c                 |

|CB E2        |set 4,d          |set 4,.d                 |

|CB E3        |set 4,e          |set 4,.e                 |

|CB E4        |set 4,h          |set 4,.h                 |

|CB E5        |set 4,l          |set 4,.l                 |

|CB E6        |set 4,(hl)       |set 4,[.hl]              |

|CB E7        |set 4,a          |set 4,.a                 |

|CB E8        |set 5,b          |set 5,.b                 |

|CB E9        |set 5,c          |set 5,.c                 |

|CB EA        |set 5,d          |set 5,.d                 |

|CB EB        |set 5,e          |set 5,.e                 |

|CB EC        |set 5,h          |set 5,.h                 |

|CB ED        |set 5,l          |set 5,.l                 |

|CB EE        |set 5,(hl)       |set 5,[.hl]              |

|CB EF        |set 5,a          |set 5,.a                 |

+-------------+-----------------+-------------------------+

|CB F0        |set 6,b          |set 6,.b                 |

|CB F1        |set 6,c          |set 6,.c                 |

|CB F2        |set 6,d          |set 6,.d                 |

|CB F3        |set 6,e          |set 6,.e                 |

|CB F4        |set 6,h          |set 6,.h                 |

|CB F5        |set 6,l          |set 6,.l                 |

|CB F6        |set 6,(hl)       |set 6,[.hl]              |

|CB F7        |set 6,a          |set 6,.a                 |

|CB F8        |set 7,b          |set 7,.b                 |

|CB F9        |set 7,c          |set 7,.c                 |

|CB FA        |set 7,d          |set 7,.d                 |

|CB FB        |set 7,e          |set 7,.e                 |

|CB FC        |set 7,h          |set 7,.h                 |

|CB FD        |set 7,l          |set 7,.l                 |

|CB FE        |set 7,(hl)       |set 7,[.hl]              |

|CB FF        |set 7,a          |set 7,.a                 |

+=============+=================+=========================+

|CC nnl nnh   |call z,nn        |call z,nn                |

|CD nnl nnh   |call nn          |call nn                  |

|CE n         |adc a,n          |addc .a,n                |

|CF           |rst 08H          |brk 08H                  |

+-------------+-----------------+-------------------------+

|D0           |ret nc           |ret nc                   |

|D1           |pop de           |pop .de                  |

|D2 nnl nnh   |jp nc,nn         |bnc nn                   |

|D3 n         |out (n),a        |out [n],.a               |

|D4 nnl nnh   |call nc,nn       |call nc,nn               |

|D5           |push de          |push .de                 |

|D6 n         |sub n            |sub .a,n                 |

|D7           |rst 10H          |brk 10H                  |

|D8           |ret c            |ret c                    |

|D9           |exx              |xchx                     |

|DA nnl nnh   |jp c,nn          |bc nn                    |

|DB n         |in a,(n)         |in .a,[n]                |

|DC nnl nnh   |call c,nn        |call c,nn                |

+=============+=================+=========================+

|DD 00        |                                           |

|.............|             invalid op-codes              |

|DD 08        |                                           |

+-------------+-----------------+-------------------------+

|DD 09        |add ix,bc        |add .ix,.bc              |

+-------------+-----------------+-------------------------+

|DD 0A        |                                           |

|.............|             invalid op-codes              |

|DD 18        |                                           |

+-------------+-----------------+-------------------------+

|DD 19        |add ix,de        |add .ix,.de              |

+-------------+-----------------+-------------------------+

|DD 1A        |                                           |

|.............|             invalid op-codes              |

|DD 20        |                                           |

+-------------+-----------------+-------------------------+

|DD 21 nnl nnh|ld ix,nn         |ld .ix,nn                |

|DD 22 nnl nnh|ld (nn),ix       |ld [nn],.ix              |

|DD 23        |inc ix           |inc .ix                  |

|DD 24        | invalid op-code |inc .ixh                 |

|DD 25        | invalid op-code |dec .ixh                 |

|DD 26 n      | invalid op-code |ld .ixh,n                |

+-------------+-----------------+-------------------------+

|DD 27        |             invalid op-code               |

|DD 28        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|DD 29        |add ix,ix        |add .ix,.ix              |

|DD 2A nnl nnh|ld ix,(nn)       |ld .ix,[nn]              |

|DD 2B        |dec ix           |dec .ix                  |

|DD 2C        | invalid op-code |inc .ixl                 |

|DD 2D        | invalid op-code |dec .ixl                 |

|DD 2E n      | invalid op-code |ld .ixl,n                |

+-------------+-----------------+-------------------------+

|DD 2F        |                                           |

|.............|             invalid op-codes              |

|DD 33        |                                           |

+-------------+-----------------+-------------------------+

|DD 34 d      |inc (ix+d)       |inc [.ix+d]              |

|DD 35 d      |dec (ix+d)       |dec [.ix+d]              |

|DD 36 d n    |ld (ix+d),n      |ld [.ix+d],n             |

+-------------+-----------------+-------------------------+

|DD 37        |             invalid op-code               |

|DD 38        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|DD 39        |add ix,sp        |add .ix,.sp              |

+-------------+-----------------+-------------------------+

|DD 40        | invalid op-code |ld .b,.b                 |

|DD 41        | invalid op-code |ld .b,.c                 |

|DD 42        | invalid op-code |ld .b,.d                 |

|DD 43        | invalid op-code |ld .b,.e                 |

|DD 44        | invalid op-code |ld .b,.ixh               |

|DD 45        | invalid op-code |ld .b,.ixl               |

|DD 46 d      |ld b,(ix+d)      |ld .b,[.ix+d]            |

|DD 47        | invalid op-code |ld .b,.a                 |

|DD 48        | invalid op-code |ld .c,.b                 |

|DD 49        | invalid op-code |ld .c,.c                 |

|DD 4A        | invalid op-code |ld .c,.d                 |

|DD 4B        | invalid op-code |ld .c,.e                 |

|DD 4C        | invalid op-code |ld .c,.ixh               |

|DD 4D        | invalid op-code |ld .c,.ixl               |

|DD 4E d      |ld c,(ix+d)      |ld .c,[.ix+d]            |

|DD 4F        | invalid op-code |ld .c,.a                 |

+-------------+-----------------+-------------------------+

|DD 50        | invalid op-code |ld .d,.b                 |

|DD 51        | invalid op-code |ld .d,.c                 |

|DD 52        | invalid op-code |ld .d,.d                 |

|DD 53        | invalid op-code |ld .d,.e                 |

|DD 54        | invalid op-code |ld .d,.ixh               |

|DD 55        | invalid op-code |ld .d,.ixl               |

|DD 56 d      |ld d,(ix+d)      |ld .d,[.ix+d]            |

|DD 57        | invalid op-code |ld .d,.a                 |

|DD 58        | invalid op-code |ld .e,.b                 |

|DD 59        | invalid op-code |ld .e,.c                 |

|DD 5A        | invalid op-code |ld .e,.d                 |

|DD 5B        | invalid op-code |ld .e,.e                 |

|DD 5C        | invalid op-code |ld .e,.ixh               |

|DD 5D        | invalid op-code |ld .e,.ixl               |

|DD 5E d      |ld e,(ix+d)      |ld .e,[.ix+d]            |

|DD 5F        | invalid op-code |ld .e,.a                 |

+-------------+-----------------+-------------------------+

|DD 60        | invalid op-code |ld .ixh,.b               |

|DD 61        | invalid op-code |ld .ixh,.c               |

|DD 62        | invalid op-code |ld .ixh,.d               |

|DD 63        | invalid op-code |ld .ixh,.e               |

|DD 64        | invalid op-code |ld .ixh,.ixh             |

|DD 65        | invalid op-code |ld .ixh,.ixl             |

|DD 66 d      |ld h,(ix+d)      |ld .h,[.ix+d]            |

|DD 67        | invalid op-code |ld .ixh,.a               |

|DD 68        | invalid op-code |ld .ixl,.b               |

|DD 69        | invalid op-code |ld .ixl,.c               |

|DD 6A        | invalid op-code |ld .ixl,.d               |

|DD 6B        | invalid op-code |ld .ixl,.e               |

|DD 6C        | invalid op-code |ld .ixl,.ixh             |

|DD 6D        | invalid op-code |ld .ixl,.ixl             |

|DD 6E d      |ld l,(ix+d)      |ld .l,[.ix+d]            |

|DD 6F        | invalid op-code |ld .ixl,.a               |

+-------------+-----------------+-------------------------+

|DD 70 d      |ld (ix+d),b      |ld [.ix+d],.b            |

|DD 71 d      |ld (ix+d),c      |ld [.ix+d],.c            |

|DD 72 d      |ld (ix+d),d      |ld [.ix+d],.d            |

|DD 73 d      |ld (ix+d),e      |ld [.ix+d],.e            |

|DD 74 d      |ld (ix+d),h      |ld [.ix+d],.h            |

|DD 75 d      |ld (ix+d),l      |ld [.ix+d],.l            |

|DD 77 d      |ld (ix+d),a      |ld [.ix+d],.a            |

|DD 78        | invalid op-code |ld .a,.b                 |

|DD 79        | invalid op-code |ld .a,.c                 |

|DD 7A        | invalid op-code |ld .a,.d                 |

|DD 7B        | invalid op-code |ld .a,.e                 |

|DD 7C        | invalid op-code |ld .a,.ixh               |

|DD 7D        | invalid op-code |ld .a,.ixl               |

|DD 7E d      |ld a,(ix+d)      |ld .a,[.ix+d]            |

|DD 7F        | invalid op-code |ld .a,.a                 |

+-------------+-----------------+-------------------------+

|DD 80        |                                           |

|.............|             invalid op-codes              |

|DD 83        |                                           |

+-------------+-----------------+-------------------------+

|DD 84        | invalid op-code |add .a,.ixh              |

|DD 85        | invalid op-code |add .a,.ixl              |

|DD 86 d      |add a,(ix+d)     |add .a,[.ix+d]           |

+-------------+-----------------+-------------------------+

|DD 87        |                                           |

|.............|             invalid op-codes              |

|DD 8B        |                                           |

+-------------+-----------------+-------------------------+

|DD 8C        | invalid op-code |addc .a,.ixh             |

|DD 8D        | invalid op-code |addc .a,.ixl             |

|DD 8E d      |adc a,(ix+d)     |addc .a,[.ix+d]          |

+-------------+-----------------+-------------------------+

|DD 8F        |                                           |

|.............|             invalid op-codes              |

|DD 83        |                                           |

+-------------+-----------------+-------------------------+

|DD 94        | invalid op-code |sub .a,.ixh              |

|DD 95        | invalid op-code |sub .a,.ixl              |

|DD 96 d      |sub (ix+d)       |sub .a,[.ix+d]           |

+-------------+-----------------+-------------------------+

|DD 97        |                                           |

|.............|             invalid op-codes              |

|DD 9B        |                                           |

+-------------+-----------------+-------------------------+

|DD 9C        | invalid op-code |subc .a,.ixh             |

|DD 9D        | invalid op-code |subc .a,.ixl             |

|DD 9E d      |sbc a,(ix+d)     |subc .a,[.ix+d]          |

+-------------+-----------------+-------------------------+

|DD 9F        |                                           |

|.............|             invalid op-codes              |

|DD A3        |                                           |

+-------------+-----------------+-------------------------+

|DD A4        | invalid op-code |and .a,.ixh              |

|DD A5        | invalid op-code |and .a,.ixl              |

|DD A6 d      |and (ix+d)       |and .a,[.ix+d]           |

+-------------+-----------------+-------------------------+

|DD A7        |                                           |

|.............|             invalid op-codes              |

|DD AB        |                                           |

+-------------+-----------------+-------------------------+

|DD AC        | invalid op-code |xor .a,.ixh              |

|DD AD        | invalid op-code |xor .a,.ixl              |

|DD AE d      |xor (ix+d)       |xor .a,[.ix+d]           |

+-------------+-----------------+-------------------------+

|DD AF        |                                           |

|.............|             invalid op-codes              |

|DD B3        |                                           |

+-------------+-----------------+-------------------------+

|DD B4        | invalid op-code |or .a,.ixh               |

|DD B5        | invalid op-code |or .a,.ixl               |

|DD B6 d      |or (ix+d)        |or .a,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD B7        |                                           |

|.............|             invalid op-codes              |

|DD BB        |                                           |

+-------------+-----------------+-------------------------+

|DD BC        | invalid op-code |cmp .a,.ixh              |

|DD BD        | invalid op-code |cmp .a,.ixl              |

|DD BE d      |cp (ix+d)        |cmp .a,[.ix+d]           |

+-------------+-----------------+-------------------------+

|DD BF        |                                           |

|.............|             invalid op-codes              |

|DD CA        |                                           |

+=============+=================+=========================+

|DD CB d 06   |rlc (ix+d)       |rol [.ix+d]              |

|DD CB d 0E   |rrc (ix+d)       |ror [.ix+d]              |

|DD CB d 16   |rl (ix+d)        |rolc [.ix+d]             |

|DD CB d 1E   |rr (ix+d)        |rorc [.ix+d]             |

+-------------+-----------------+-------------------------+

|DD CB d 1F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 25   |                                           |

+-------------+-----------------+------------+------------+

|DD CB d 26   |sla (ix+d)       |shl [.ix+d] |shla [.ix+d]|

+-------------+-----------------+------------+------------+

|DD CB d 27   |                                           |

|.............|             invalid op-codes              |

|DD CB d 2D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 2E   |sra (ix+d)       |shra [ix+d]              |

+-------------+-----------------+-------------------------+

|DD CB d 2F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 3D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 3E   |srl (ix+d)       |shr [.ix+d]              |

+-------------+-----------------+-------------------------+

|DD CB d 3F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 45   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 46   |bit 0,(ix+d)     |bit 0,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 47   |                                           |

|.............|             invalid op-codes              |

|DD CB d 4D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 4E   |bit 1,(ix+d)     |bit 1,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 4F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 55   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 56   |bit 2,(ix+d)     |bit 2,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 57   |                                           |

|.............|             invalid op-codes              |

|DD CB d 5D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 5E   |bit 3,(ix+d)     |bit 3,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 5F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 65   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 66   |bit 4,(ix+d)     |bit 4,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 67   |                                           |

|.............|             invalid op-codes              |

|DD CB d 6D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 6E   |bit 5,(ix+d)     |bit 5,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 6F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 75   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 76   |bit 6,(ix+d)     |bit 6,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 77   |                                           |

|.............|             invalid op-codes              |

|DD CB d 7D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 7E   |bit 7,(ix+d)     |bit 7,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 7F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 85   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 86   |res 0,(ix+d)     |clr 0,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 87   |                                           |

|.............|             invalid op-codes              |

|DD CB d 8D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 8E   |res 1,(ix+d)     |clr 1,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 8F   |                                           |

|.............|             invalid op-codes              |

|DD CB d 95   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 96   |res 2,(ix+d)     |clr 2,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 97   |                                           |

|.............|             invalid op-codes              |

|DD CB d 9D   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d 9E   |res 3,(ix+d)     |clr 3,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d 9F   |                                           |

|.............|             invalid op-codes              |

|DD CB d A5   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d A6   |res 4,(ix+d)     |clr 4,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d A7   |                                           |

|.............|             invalid op-codes              |

|DD CB d AD   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d AE   |res 5,(ix+d)     |clr 5,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d AF   |                                           |

|.............|             invalid op-codes              |

|DD CB d B5   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d B6   |res 6,(ix+d)     |clr 6,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d B7   |                                           |

|.............|             invalid op-codes              |

|DD CB d BD   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d BE   |res 7,(ix+d)     |clr 7,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d BF   |                                           |

|.............|             invalid op-codes              |

|DD CB d C5   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d C6   |set 0,(ix+d)     |set 0,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d C7   |                                           |

|.............|             invalid op-codes              |

|DD CB d CD   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d CE   |set 1,(ix+d)     |set 1,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d CF   |                                           |

|.............|             invalid op-codes              |

|DD CB d D5   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d D6   |set 2,(ix+d)     |set 2,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d D7   |                                           |

|.............|             invalid op-codes              |

|DD CB d DD   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d DE   |set 3,(ix+d)     |set 3,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d DF   |                                           |

|.............|             invalid op-codes              |

|DD CB d E5   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d E6   |set 4,(ix+d)     |set 4,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d E7   |                                           |

|.............|             invalid op-codes              |

|DD CB d ED   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d EE   |set 5,(ix+d)     |set 5,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d EF   |                                           |

|.............|             invalid op-codes              |

|DD CB d F5   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d F6   |set 6,(ix+d)     |set 6,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d F7   |                                           |

|.............|             invalid op-codes              |

|DD CB d FD   |                                           |

+-------------+-----------------+-------------------------+

|DD CB d FE   |set 7,(ix+d)     |set 7,[.ix+d]            |

+-------------+-----------------+-------------------------+

|DD CB d FF   |             invalid op-code               |

+=============+=================+=========================+

|DD CC        |                                           |

|.............|             invalid op-codes              |

|DD E0        |                                           |

+-------------+-----------------+-------------------------+

|DD E1        |pop ix           |pop .ix                  |

+-------------+-----------------+-------------------------+

|DD E2        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|DD E3        |ex (sp),ix       |xch [.sp],.ix            |

+-------------+-----------------+-------------------------+

|DD E4        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|DD E5        |push ix          |push .ix                 |

+-------------+-----------------+-------------------------+

|DD E6        |                                           |

|.............|             invalid op-codes              |

|DD E8        |                                           |

+-------------+-----------------+-------------------------+

|DD E9        |jp (ix)          |br [.ix]                 |

+-------------+-----------------+-------------------------+

|DD EA        |                                           |

|.............|             invalid op-codes              |

|DD F8        |                                           |

+-------------+-----------------+-------------------------+

|DD F9        |ld sp,ix         |ld .sp,.ix               |

+-------------+-----------------+-------------------------+

|DD FA        |                                           |

|.............|             invalid op-codes              |

|DD FF        |                                           |

+=============+=================+=========================+

|DE n         |sbc a,n          |subc .a,n                |

|DF           |rst 18H          |brk 18H                  |

+-------------+-----------------+-------------------------+

|E0           |ret po           |ret po                   |

|E1           |pop hl           |pop .hl                  |

|E2 nnl nnh   |jp po,nn         |bpo nn                   |

|E3           |ex (sp),hl       |xch [.sp],.hl            |

|E4 nnl nnh   |call po,nn       |call po,nn               |

|E5           |push hl          |push .hl                 |

|E6 n         |and n            |and .a,n                 |

|E7           |rst 20H          |brk 20H                  |

|E8           |ret pe           |ret pe                   |

|E9           |jp (hl)          |br [.hl]                 |

|EA nnl nnh   |jp pe,nn         |bpe nn                   |

|EB           |ex de,hl         |xch .de,.hl              |

|EC nnl nnh   |call pe,nn       |call pe,nn               |

+=============+=================+=========================+

|ED 00        |                                           |

|.............|             invalid op-codes              |

|ED 3F        |                                           |

+-------------+-----------------+-------------------------+

|ED 40        |in b,(c)         |in .b,[.c]               |

|ED 41        |out (c),b        |out [.c],.b              |

|ED 42        |sbc hl,bc        |subc .hl,.bc             |

|ED 43 nnl nnh|ld (nn),bc       |ld [nn],.bc              |

|ED 44        |neg              |neg .a                   |

|ED 45        |retn             |retn                     |

|ED 46        |im 0             |im 0                     |

|ED 47        |ld i,a           |ld .i,.a                 |

|ED 48        |in c,(c)         |in .c,[.c]               |

|ED 49        |out (c),c        |out [.c],.c              |

|ED 4A        |adc hl,bc        |addc .hl,.bc             |

|ED 4B nnl nnh|ld bc,(nn)       |ld .bc,[nn]              |

+-------------+-----------------+-------------------------+

|ED 4C        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED 4D        |reti             |reti                     |

+-------------+-----------------+-------------------------+

|ED 4E        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED 4F        |ld r,a           |ld .r,.a                 |

+-------------+-----------------+-------------------------+

|ED 50        |in d,(c)         |in .d,[.c]               |

|ED 51        |out (c),d        |out [.c],.d              |

|ED 52        |sbc hl,de        |subc .hl,.de             |

|ED 53 nnl nnh|ld (nn),de       |ld [nn],.de              |

+-------------+-----------------+-------------------------+

|ED 54        |             invalid op-code               |

|ED 55        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED 56        |im 1             |im 1                     |

|ED 57        |ld a,i           |ld .a,.i                 |

|ED 58        |in e,(c)         |in .e,[.c]               |

|ED 59        |out (c),e        |out [.c],.e              |

|ED 5A        |adc hl,de        |addc .hl,.de             |

|ED 5B nnl nnh|ld de,(nn)       |ld .de,[nn]              |

+-------------+-----------------+-------------------------+

|ED 5C        |             invalid op-code               |

|ED 5D        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED 5E        |im 2             |im 2                     |

|ED 5F        |ld a,r           |ld .a,.r                 |

+-------------+-----------------+-------------------------+

|ED 60        |in h,(c)         |in .h,[.c]               |

|ED 61        |out (c),h        |out [.c],.h              |

|ED 62        |sbc hl,hl        |subc .hl,.hl             |

|ED 63 nnl nnh|ld (nn),hl       |ld [nn],.hl              |

+-------------+-----------------+-------------------------+

|ED 64        |                                           |

|.............|             invalid op-codes              |

|ED 66        |                                           |

+-------------+-----------------+-------------------------+

|ED 67        |rrd              |rol4 [.hl]               |

|ED 68        |in l,(c)         |in .l,[.c]               |

|ED 69        |out (c),l        |out [.c],.l              |

|ED 6A        |adc hl,hl        |addc .hl,.hl             |

|ED 6B nnl nnh|ld hl,(nn)       |ld .hl,[nn]              |

+-------------+-----------------+-------------------------+

|ED 6C        |                                           |

|.............|             invalid op-codes              |

|ED 6E        |                                           |

+-------------+-----------------+-------------------------+

|ED 6F        |rld              |rol4 [.hl]               |

+-------------+-----------------+-------------------------+

|ED 70        | *1              |in .f,[.c]               |

+-------------+-----------------+-------------------------+

|ED 71        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED 72        |sbc hl,sp        |subc .hl,.sp             |

|ED 73 nnl nnh|ld (nn),sp       |ld [nn],.sp              |

+-------------+-----------------+-------------------------+

|ED 74        |                                           |

|.............|             invalid op-codes              |

|ED 77        |                                           |

+-------------+-----------------+-------------------------+

|ED 78        |in a,(c)         |in .a,[.c]               |

|ED 79        |out (c),a        |out [.c],.a              |

|ED 7A        |adc hl,sp        |addc .hl,.sp             |

|ED 7B nnl nnh|ld sp,(nn)       |ld .sp,[nn]              |

+-------------+-----------------+-------------------------+

|ED 7C        |                                           |

|.............|             invalid op-codes              |

|ED 9F        |                                           |

+-------------+-----------------+-------------------------+

|ED A0        |ldi              |move [.hl++],[.de++]     |

|ED A1        |cpi              |cmp .a,[.hl++]           |

|ED A2        |ini              |in [.hl++],[.c]          |

|ED A3        |outi             |out [.c],[.hl++]         |

+-------------+-----------------+-------------------------+

|ED A4        |                                           |

|.............|             invalid op-codes              |

|ED A7        |                                           |

+-------------+-----------------+-------------------------+

|ED A8        |ldd              |move [.hl--],[.de--]     |

|ED A9        |cpd              |cmp .a,[.hl--]           |

|ED AA        |ind              |in [.hl--],[.c]          |

|ED AB        |outd             |out [.c],[.hl--]         |

+-------------+-----------------+-------------------------+

|ED AC        |                                           |

|.............|             invalid op-codes              |

|ED AF        |                                           |

+-------------+-----------------+-------------------------+

|ED B0        |ldir             |movem [.hl++],[.de++]    |

|ED B1        |cpir             |cmpm .a,[.hl++]          |

|ED B2        |inir             |inm [.hl++],[.c]         |

|ED B3        |otir             |outm [.c],[.hl++]        |

+-------------+-----------------+-------------------------+

|ED B4        |                                           |

|.............|             invalid op-codes              |

|ED B7        |                                           |

+-------------+-----------------+-------------------------+

|ED B8        |lddr             |movem [.hl--],[.de--]    |

|ED B9        |cpdr             |cmpm .a,[.hl--]          |

|ED BA        |indr             |inm [.hl--],[.c]         |

|ED BB        |otdr             |outm [.c],[.hl--]        |

+-------------+-----------------+-------------------------+

|ED BC        |                                           |

|.............|             invalid op-codes              |

|ED C0        |                                           |

+-------------+-----------------+-------------------------+

|ED C1        | invalid op-code |mulub .a,.b              |

+-------------+-----------------+-------------------------+

|ED C2        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED C3        | invalid op-code |muluw .hl,.bc            |

+-------------+-----------------+-------------------------+

|ED C4        |                                           |

|.............|             invalid op-codes              |

|ED C8        |                                           |

+-------------+-----------------+-------------------------+

|ED C9        | invalid op-code |mulub .a,.c              |

+-------------+-----------------+-------------------------+

|ED CA        |                                           |

|.............|             invalid op-codes              |

|ED D0        |                                           |

+-------------+-----------------+-------------------------+

|ED D1        | invalid op-code |mulub .a,.d              |

+-------------+-----------------+-------------------------+

|ED C2        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED D3        | invalid op-code |muluw .hl,.de            |

+-------------+-----------------+-------------------------+

|ED D4        |                                           |

|.............|             invalid op-codes              |

|ED D8        |                                           |

+-------------+-----------------+-------------------------+

|ED D9        | invalid op-code |mulub .a,.e              |

+-------------+-----------------+-------------------------+

|ED DA        |                                           |

|.............|             invalid op-codes              |

|ED E0        |                                           |

+-------------+-----------------+-------------------------+

|ED E1        | invalid op-code |mulub .a,.h              |

+-------------+-----------------+-------------------------+

|ED E2        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|ED E3        | invalid op-code |muluw .hl,.hl            |

+-------------+-----------------+-------------------------+

|ED E4        |                                           |

|.............|             invalid op-codes              |

|ED E8        |                                           |

+-------------+-----------------+-------------------------+

|ED E9        | invalid op-code |mulub .a,.l              |

+-------------+-----------------+-------------------------+

|ED EA        |                                           |

|.............|             invalid op-codes              |

|ED F2        |                                           |

+-------------+-----------------+-------------------------+

|ED F3        | invalid op-code |muluw .hl,.sp            |

+-------------+-----------------+-------------------------+

|ED F4        |                                           |

|.............|             invalid op-codes              |

|ED F8        |                                           |

+-------------+-----------------+-------------------------+

|ED F9        | invalid op-code |mulub .a,.a              |

+-------------+-----------------+-------------------------+

|ED FA        |                                           |

|.............|             invalid op-codes              |

|ED FF        |                                           |

+=============+=================+=========================+

|EE n         |xor n            |xor .a,n                 |

|EF           |rst 28H          |brk 28H                  |

|F0           |ret p            |ret p                    |

|F1           |pop af           |pop .af                  |

|F2 nnl nnh   |jp p,nn          |bp nn                    |

|F3           |di               |di                       |

|F4 nnl nnh   |call p,nn        |call p,nn                |

|F5           |push af          |push .af                 |

|F6 n         |or n             |or .a,n                  |

|F7           |rst 30H          |brk 30H                  |

|F8           |ret m            |ret m                    |

|F9           |ld sp,hl         |ld .sp,.hl               |

|FA nnl nnh   |jp m,nn          |bm nn                    |

|FB           |ei               |ei                       |

|FC nnl nnh   |call m,nn        |call m,nn                |

+=============+=================+=========================+

|FD 00        |                                           |

|.............|             invalid op-codes              |

|FD 08        |                                           |

+-------------+-----------------+-------------------------+

|FD 09        |add iy,bc        |add .iy,.bc              |

+-------------+-----------------+-------------------------+

|FD 0A        |                                           |

|.............|             invalid op-codes              |

|FD 18        |                                           |

+-------------+-----------------+-------------------------+

|FD 19        |add iy,de        |add .iy,.de              |

+-------------+-----------------+-------------------------+

|FD 1A        |                                           |

|.............|             invalid op-codes              |

|FD 20        |                                           |

+-------------+-----------------+-------------------------+

|FD 21 nnl nnh|ld iy,nn         |ld .iy,nn                |

|FD 22 nnl nnh|ld (nn),iy       |ld [nn],.iy              |

|FD 23        |inc iy           |inc .iy                  |

|FD 24        | invalid op-code |inc .iyh                 |

|FD 25        | invalid op-code |dec .iyh                 |

|FD 26 n      | invalid op-code |ld .iyh,n                |

+-------------+-----------------+-------------------------+

|FD 27        |             invalid op-code               |

|FD 28        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|FD 29        |add iy,iy        |add .iy,.iy              |

|FD 2A nnl nnh|ld iy,(nn)       |ld .iy,[nn]              |

|FD 2B        |dec iy           |dec .iy                  |

|FD 2C        | invalid op-code |inc .iyl                 |

|FD 2D        | invalid op-code |dec .iyl                 |

|FD 2E n      | invalid op-code |ld .iyl,n                |

+-------------+-----------------+-------------------------+

|FD 2F        |                                           |

|.............|             invalid op-codes              |

|FD 33        |                                           |

+-------------+-----------------+-------------------------+

|FD 34 d      |inc (iy+d)       |inc [.iy+d]              |

|FD 35 d      |dec (iy+d)       |dec [.iy+d]              |

|FD 36 d n    |ld (iy+d),n      |ld [.iy+d],n             |

+-------------+-----------------+-------------------------+

|FD 37        |             invalid op-code               |

|FD 38        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|FD 39        |add iy,sp        |add .iy,.sp              |

+-------------+-----------------+-------------------------+

|FD 40        | invalid op-code |ld .b,.b                 |

|FD 41        | invalid op-code |ld .b,.c                 |

|FD 42        | invalid op-code |ld .b,.d                 |

|FD 43        | invalid op-code |ld .b,.e                 |

|FD 44        | invalid op-code |ld .b,.iyh               |

|FD 45        | invalid op-code |ld .b,.iyl               |

|FD 46 d      |ld b,(iy+d)      |ld .b,[.iy+d]            |

|FD 47        | invalid op-code |ld .b,.a                 |

|FD 48        | invalid op-code |ld .c,.b                 |

|FD 49        | invalid op-code |ld .c,.c                 |

|FD 4A        | invalid op-code |ld .c,.d                 |

|FD 4B        | invalid op-code |ld .c,.e                 |

|FD 4C        | invalid op-code |ld .c,.iyh               |

|FD 4D        | invalid op-code |ld .c,.iyl               |

|FD 4E d      |ld c,(iy+d)      |ld .c,[.iy+d]            |

|FD 4F        | invalid op-code |ld .c,.a                 |

+-------------+-----------------+-------------------------+

|FD 50        | invalid op-code |ld .d,.b                 |

|FD 51        | invalid op-code |ld .d,.c                 |

|FD 52        | invalid op-code |ld .d,.d                 |

|FD 53        | invalid op-code |ld .d,.e                 |

|FD 54        | invalid op-code |ld .d,.iyh               |

|FD 55        | invalid op-code |ld .d,.iyl               |

|FD 56 d      |ld d,(iy+d)      |ld .d,[.iy+d]            |

|FD 57        | invalid op-code |ld .d,.a                 |

|FD 58        | invalid op-code |ld .e,.b                 |

|FD 59        | invalid op-code |ld .e,.c                 |

|FD 5A        | invalid op-code |ld .e,.d                 |

|FD 5B        | invalid op-code |ld .e,.e                 |

|FD 5C        | invalid op-code |ld .e,.iyh               |

|FD 5D        | invalid op-code |ld .e,.iyl               |

|FD 5E d      |ld e,(iy+d)      |ld .e,[.iy+d]            |

|FD 5F        | invalid op-code |ld .e,.a                 |

+-------------+-----------------+-------------------------+

|FD 60        | invalid op-code |ld .iyh,.b               |

|FD 61        | invalid op-code |ld .iyh,.c               |

|FD 62        | invalid op-code |ld .iyh,.d               |

|FD 63        | invalid op-code |ld .iyh,.e               |

|FD 64        | invalid op-code |ld .iyh,.iyh             |

|FD 65        | invalid op-code |ld .iyh,.iyl             |

|FD 66 d      |ld h,(iy+d)      |ld .h,[.iy+d]            |

|FD 67        | invalid op-code |ld .iyh,.a               |

|FD 68        | invalid op-code |ld .iyl,.b               |

|FD 69        | invalid op-code |ld .iyl,.c               |

|FD 6A        | invalid op-code |ld .iyl,.d               |

|FD 6B        | invalid op-code |ld .iyl,.e               |

|FD 6C        | invalid op-code |ld .iyl,.iyh             |

|FD 6D        | invalid op-code |ld .iyl,.iyl             |

|FD 6E d      |ld l,(iy+d)      |ld .l,[.iy+d]            |

|FD 6F        | invalid op-code |ld .iyl,.a               |

+-------------+-----------------+-------------------------+

|FD 70 d      |ld (iy+d),b      |ld [.iy+d],.b            |

|FD 71 d      |ld (iy+d),c      |ld [.iy+d],.c            |

|FD 72 d      |ld (iy+d),d      |ld [.iy+d],.d            |

|FD 73 d      |ld (iy+d),e      |ld [.iy+d],.e            |

|FD 74 d      |ld (iy+d),h      |ld [.iy+d],.h            |

|FD 75 d      |ld (iy+d),l      |ld [.iy+d],.l            |

+-------------+-----------------+-------------------------+

|FD 76        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|FD 77 d      |ld (iy+d),a      |ld [.iy+d],.a            |

|FD 78        | invalid op-code |ld .a,.b                 |

|FD 79        | invalid op-code |ld .a,.c                 |

|FD 7A        | invalid op-code |ld .a,.d                 |

|FD 7B        | invalid op-code |ld .a,.e                 |

|FD 7C        | invalid op-code |ld .a,.iyh               |

|FD 7D        | invalid op-code |ld .a,.iyl               |

|FD 7E d      |ld a,(iy+d)      |ld .a,[.iy+d]            |

|FD 7F        | invalid op-code |ld .a,.a                 |

+-------------+-----------------+-------------------------+

|FD 80        |                                           |

|.............|             invalid op-codes              |

|FD 83        |                                           |

+-------------+-----------------+-------------------------+

|FD 84        | invalid op-code |add .a,.iyh              |

|FD 85        | invalid op-code |add .a,.iyl              |

|FD 86 d      |add a,(iy+d)     |add .a,[.iy+d]           |

+-------------+-----------------+-------------------------+

|FD 87        |                                           |

|.............|             invalid op-codes              |

|FD 8B        |                                           |

+-------------+-----------------+-------------------------+

|FD 8C        | invalid op-code |addc .a,.iyh             |

|FD 8D        | invalid op-code |addc .a,.iyl             |

|FD 8E d      |adc a,(iy+d)     |addc .a,[.iy+d]          |

+-------------+-----------------+-------------------------+

|FD 8F        |                                           |

|.............|             invalid op-codes              |

|FD 93        |                                           |

+-------------+-----------------+-------------------------+

|FD 94        | invalid op-code |sub .a,.iyh              |

|FD 95        | invalid op-code |sub .a,.iyl              |

|FD 96 d      |sub (iy+d)       |sub .a,[.iy+d]           |

+-------------+-----------------+-------------------------+

|FD 97        |                                           |

|.............|             invalid op-codes              |

|FD 9B        |                                           |

+-------------+-----------------+-------------------------+

|FD 9C        | invalid op-code |subc .a,.iyh             |

|FD 9D        | invalid op-code |subc .a,.iyl             |

|FD 9E d      |sbc a,(iy+d)     |subc .a,[.iy+d]          |

+-------------+-----------------+-------------------------+

|FD 9F        |                                           |

|.............|             invalid op-codes              |

|FD A3        |                                           |

+-------------+-----------------+-------------------------+

|FD A4        | invalid op-code |and .a,.iyh              |

|FD A5        | invalid op-code |and .a,.iyl              |

|FD A6 d      |and (iy+d)       |and .a,[.iy+d]           |

+-------------+-----------------+-------------------------+

|FD A7        |                                           |

|.............|             invalid op-codes              |

|FD AB        |                                           |

+-------------+-----------------+-------------------------+

|FD AC        | invalid op-code |xor .a,.iyh              |

|FD AD        | invalid op-code |xor .a,.iyl              |

|FD AE d      |xor (iy+d)       |xor .a,[.iy+d]           |

+-------------+-----------------+-------------------------+

|FD AF        |                                           |

|.............|             invalid op-codes              |

|FD B3        |                                           |

+-------------+-----------------+-------------------------+

|FD B4        | invalid op-code |or .a,.iyh               |

|FD B5        | invalid op-code |or .a,.iyl               |

|FD B6 d      |or (iy+d)        |or .a,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD B7        |                                           |

|.............|             invalid op-codes              |

|FD BB        |                                           |

+-------------+-----------------+-------------------------+

|FD BC        | invalid op-code |cmp .a,.iyh              |

|FD BD        | invalid op-code |cmp .a,.iyl              |

|FD BE d      |cp (iy+d)        |cmp .a,[.iy+d]           |

+-------------+-----------------+-------------------------+

|FD BF        |                                           |

|.............|                                           |

|FD CA        |             invalid op-codes              |

|FD CB d 00   |                                           |

|.............|                                           |

|FD CB d 05   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 06   |rlc (iy+d)       |rol [.iy+d]              |

+-------------+-----------------+-------------------------+

|FD CB d 07   |                                           |

|.............|             invalid op-codes              |

|FD CB d 0D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 0E   |rrc (iy+d)       |ror [.iy+d]              |

+-------------+-----------------+-------------------------+

|FD CB d 0F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 15   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 16   |rl (iy+d)        |rolc [.iy+d]             |

+-------------+-----------------+-------------------------+

|FD CB d 17   |                                           |

|.............|             invalid op-codes              |

|FD CB d 1D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 1E   |rr (iy+d)        |rorc [.iy+d]             |

+-------------+-----------------+-------------------------+

|FD CB d 1F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 25   |                                           |

+-------------+-----------------+------------+------------+

|FD CB d 26   |sla (iy+d)       |shl [.iy+d] |shla [.iy+d]|

+-------------+-----------------+------------+------------+

|FD CB d 27   |                                           |

|.............|             invalid op-codes              |

|FD CB d 2D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 2E   |sra (iy+d)       |shra [iy+d]              |

+-------------+-----------------+-------------------------+

|FD CB d 2F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 3D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 3E   |srl (iy+d)       |shr [.iy+d]              |

+-------------+-----------------+-------------------------+

|FD CB d 3F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 45   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 46   |bit 0,(iy+d)     |bit 0,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 47   |                                           |

|.............|             invalid op-codes              |

|FD CB d 4D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 4E   |bit 1,(iy+d)     |bit 1,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 4F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 55   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 56   |bit 2,(iy+d)     |bit 2,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 57   |                                           |

|.............|             invalid op-codes              |

|FD CB d 5D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 5E   |bit 3,(iy+d)     |bit 3,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 5F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 65   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 66   |bit 4,(iy+d)     |bit 4,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 67   |                                           |

|.............|             invalid op-codes              |

|FD CB d 6D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 6E   |bit 5,(iy+d)     |bit 5,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 6F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 75   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 76   |bit 6,(iy+d)     |bit 6,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 77   |                                           |

|.............|             invalid op-codes              |

|FD CB d 7D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 7E   |bit 7,(iy+d)     |bit 7,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 7F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 85   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 86   |res 0,(iy+d)     |clr 0,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 87   |                                           |

|.............|             invalid op-codes              |

|FD CB d 8D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 8E   |res 1,(iy+d)     |clr 1,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 8F   |                                           |

|.............|             invalid op-codes              |

|FD CB d 95   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 96   |res 2,(iy+d)     |clr 2,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 97   |                                           |

|.............|             invalid op-codes              |

|FD CB d 9D   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d 9E   |res 3,(iy+d)     |clr 3,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 9F   |                                           |

|.............|             invalid op-codes              |

|FD CB d A5   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d A6   |res 4,(iy+d)     |clr 4,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d A7   |                                           |

|.............|             invalid op-codes              |

|FD CB d AD   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d AE   |res 5,(iy+d)     |clr 5,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d AF   |                                           |

|.............|             invalid op-codes              |

|FD CB d B5   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d B6   |res 6,(iy+d)     |clr 6,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d B7   |                                           |

|.............|             invalid op-codes              |

|FD CB d BD   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d BE   |res 7,(iy+d)     |clr 7,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d 3F   |                                           |

|.............|             invalid op-codes              |

|FD CB d C5   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d C6   |set 0,(iy+d)     |set 0,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d C7   |                                           |

|.............|             invalid op-codes              |

|FD CB d CD   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d CE   |set 1,(iy+d)     |set 1,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d CF   |                                           |

|.............|             invalid op-codes              |

|FD CB d D5   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d D6   |set 2,(iy+d)     |set 2,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d D7   |                                           |

|.............|             invalid op-codes              |

|FD CB d DD   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d DE   |set 3,(iy+d)     |set 3,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d DF   |                                           |

|.............|             invalid op-codes              |

|FD CB d E5   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d E6   |set 4,(iy+d)     |set 4,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d E7   |                                           |

|.............|             invalid op-codes              |

|FD CB d ED   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d EE   |set 5,(iy+d)     |set 5,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d EF   |                                           |

|.............|             invalid op-codes              |

|FD CB d F5   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d F6   |set 6,(iy+d)     |set 6,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d F7   |                                           |

|.............|             invalid op-codes              |

|FD CB d FD   |                                           |

+-------------+-----------------+-------------------------+

|FD CB d FE   |set 7,(iy+d)     |set 7,[.iy+d]            |

+-------------+-----------------+-------------------------+

|FD CB d FF   |                                           |

|FD CC        |             invalid op-codes              |

|.............|                                           |

|FD E0        |                                           |

+-------------+-----------------+-------------------------+

|FD E1        |pop iy           |pop .iy                  |

+-------------+-----------------+-------------------------+

|FD E2        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|FD E3        |ex (sp),iy       |xch [.sp],.iy            |

+-------------+-----------------+-------------------------+

|FD E4        |             invalid op-code               |

+-------------+-----------------+-------------------------+

|FD E5        |push iy          |push .iy                 |

+-------------+-----------------+-------------------------+

|FD E6        |                                           |

|.............|             invalid op-codes              |

|FD E8        |                                           |

+-------------+-----------------+-------------------------+

|FD E9        |jp (iy)          |br [.iy]                 |

+-------------+-----------------+-------------------------+

|FD EA        |                                           |

|.............|             invalid op-codes              |

|FD F8        |                                           |

+-------------+-----------------+-------------------------+

|FD F9        |ld sp,iy         |ld .sp,.iy               |

+-------------+-----------------+-------------------------+

|FD FA        |                                           |

|.............|             invalid op-codes              |

|FD FF        |                                           |

+=============+=================+=========================+

|FE n         |cp n             |cmp .a,n                 |

|FF           |rst 38H          |brk 38H                  |

+-------------+-----------------+-------------------------+